Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ADVANCED DIGITAL SIGNAL PROCESSING			       Time: 3 hours

Subject Code: 12EI301				         						       Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

- Determine the circular convolution of the two given sequences x1(n) = {1, 2,3,4} and  x2(n)={ 1,1,2,2} using DFT and IDFT method.                                         				(10)

- Obtain the Inverse Z transform of  X(Z) using partial fraction method.

X(z) =1(1−z−1)(1−z−1)2.														(10)

2.	a.	Derive the Convolution Sum.                                                                              		(8)

b.	Determine whether the given system x (n + 3) is Linear, Time Invariant and  BIBO  Stable.                                                                                                      				(12)

(OR)

3.	a.	Show that x(t)  =  t u(t)  is neither an energy signal nor a power signal.             		(8)

b.	Discuss briefly the various elementary Continuous and Discrete Time Signals with relevant sketches.                                                                                                 				(12)

4.	a.	Which IIR realization structure is called Canonic realization structure? Why is it called so?

(4)

b.	Obtain the Direct Form II realization structure for the transfer function:

1 –  (1 / 2) Z -1  +  (1 / 5) Z -2  +  (5 / 27) Z -3

H(Z)  =   ---------------------------------------------------------.                          			(16)

1 +  (3 / 7) Z -1    (4 / 11) Z -2  +  (1 / 6) Z -3

(OR)

5.	a.	What is meant by Bilinear Transformation? Derive how the analog transfer function is 			transformed in to a digital transfer function using Bilinear Transformation.  			(14)

- Convert the following analog transfer function in to digital transfer function using

Bilinear Transformation. Assume T = 1 Sec.                                                      		(6)

H(s)=1s+5.

6.	a.	Realize the following transfer function with minimum number of multipliers: 		(10)

H(Z)  = 1 + (1 / 2) Z -1 + (1 / 3) Z -2 + (1 / 4) Z -3 + (1 / 3) Z -4 + (1 / 2) Z -5  + Z -6

b.	Classify the various windows in detail.                                                               		(10)

(OR)

7.	a.	What is meant by Gibb’s phenomenon? How can it be reduced?                       		(4)

b.	Design a Linear Phase FIR  filter  with a frequency response.

Hd(ejω) = e−j3ω −π4≤ω≤π40 −π4≤ω≤π

using   Hamming Window  with   N  =  7.                                                          		(16)

[P.T.O]

8.	a.	Show that the Decimation process is a frequency stretching process through derivation. 	Sketch its various spectra.                                                               					(15)

b.	Discuss the significance of Polyphase Filter Structures.                                    		(5)

(OR)

9.	a.	 Explain in detail the design of phase shifters in Multirate DSP with suitable diagrams.

(10)

b.	Show that the Upsampling system and the Downsampling systems are Time Variant Systems.                                                                                                            			(10)