Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ADVANCED PROGRAMMABLE SIGNAL PROCESSOR           Time: 3 hours

Subject Code: 12EI321					         					       Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Draw and explain the subband processing system and filter banks.					(10)

b.	Sketch the frequency spectrum of f(t) = sin 2π(1000)t + sin 2π(1250)t. Determine the 	minimum sampling rate required to reconstruct the signal from its samples.  What should 	be the cutoff frequencies of the reconstruction (smoothing) filter? If the signal is sampled 	at the rate of 2000 samples /s what would be the output of the smoothing filter?		(10)

2.	a.	Explain why the P-DSPs have multiple address and data bus for internal memory and 	peripherals but have only a single address and data bus for the external memory and 	peripherals?																	(10)

b.	Justify the need for modified bus structures.										(10)

(OR)

3.	Explain with examples about the special addressing modes in P-DSPs.

4.	Enumerate on the Central Processing Unit of TMS320C5X.

(OR)

5.	Describe about the following units of TMS320C5X:

a.	Parallel Logic Unit.															(7)

b.	On-chip memory.																(7)

c.	On-chip peripherals.															(6)

6.	Discuss about the basic addressing modes of C5X with examples.				   (5 (types)x4)

(OR)

7.	Explain:

a.	Pipeline Operation on ARAU Memory-Mapped Registers. 						(10)

b.	Pipeline Operation on External Memory Conflict. 								(10)

8.	Describe:

a.	Distributed Arithmetic Filter as a State Machine.									(10)

b.	Parallel Implementation of DA FIR Filter.										(10)

(OR)

9.	Using Multiplier blocks explain:

a.	Direct I form of IIR filter. 														(7)

b.	Direct II form of IIR filter. 													(7)

c.	Lattice filter.																	(6)