Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: SEPARATION PROCESSES IN FOOD ENGINEERING	       Time: 3 hours

Subject Code: 12FP302				         					       	       Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Discuss in detail the various devices for pressure measurement with suitable sketchs.                                                                                                                                        																					(12)

b. 	A pitot tube is to be used for measurement of the flow of steam at a pressure of 300 KPa above atmospheric pressure, flowing in a 10 cm diameter pipe. If it is desired to measure flow rates in this pipe of between 300 and 600 Kg/h, what would be the differential pressure across the tube, in mm of water.						      							(8)

2.	a. 	Explain the construction and working of plate and frame filter press with a neat sketch.

(10)

b. 	Derive the expression for constant rate and constant volume filtration.                		(10)

(OR)

3.	a. 	Explain how gravitational sedimentation is achieved in liquids.			     		(10)

b. 	Discuss with a neat sketch the working of a gravity thickener (sedimentation).    		(10)

4. 	Discuss the theory and principle of centrifugal separation and working of a bowl centrifuge.

(OR)

5. 	a. 	Derive an equation for the radius of the neutral zone in centrifugal separation of liquids.

(12)

b. 	Explain the working of a nozzle centrifuge with a neat sketch.                             		(8)

6. 	a. 	Give a detailed account of principle and types of membrane filtration.	            		(12)

b. 	Write a note on types of membrane equipments with a suitable diagram.	 			(8)

(OR)

7.	a. 	Explain in detail the configuration of flat plate membrane systems with a suitable sketch.	 																				(12)

b.	Discuss the reverse osmosis process of separation and its application in food industry.  																						(8)

8. 	a. 	Discuss in detail the theory and principle of gas absorption with a suitable example.                                                                                                                                          																					(12)

b. 	Explain the criteria for the design of gas absorption equipment.			     		(8)

(OR)

9. 	a. 	Give a detailed note on applications of diffusion and migration in food packaging. 	(10)

b. 	Explain the basics of diffusion of gases in liquid and solid foods with an example. 	(10)