Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: BASIC MECHANICAL ENGINEERING				      	       Time: 3 hours

Subject Code: 12ME103						       		    		       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is the function of a fusible plug in a boiler?

2.	What is meant by scavenging in IC engines?

3.	State the use of a Moderator in a nuclear reactor.

4.	Name the metals that can be used as an absorbing surface in a flat plate collector.

5.	Define Bulk Modulus.

6.	State Hooke’s law.

7.	What is recrystallisation temperature?

8.	Why is flux used in welding?

9.	State the purpose of flutes in a twist drill.

10.	Define Creep.

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate between impulse and reaction turbine.

12. 	State the use of a spillway in a hydroelectric power plant.

13. 	An axial load of 38 kN acts on a bar, 24 mm in diameter and 400 mm length. The bar elongates by 0.165 mm. Find the Young’s Modulus of the bar.

14.	Name the three melting furnaces and their differences.

15.	What is the difference between live centre and dead centre?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain with a neat sketch, the working of a Babcock and Wilcox boiler.

(OR)

17.	With neat sketches, explain the working of a four-stroke diesel engine.

18.	Discuss the working of a Diesel Power plant. List its merits and demerits.

(OR)

19.	Explain the working of an Ocean Thermal Energy conversion power plant.

20.	Sketch and explain the various phases of a stress strain curve of ductile materials.

(OR)

21.	Discuss the following AUTOCAD commands in detail with examples:

a.	Mirror 		b.	Chamfer 	c.	Fillet 		d.	Limits 		e.	Array.

22.	Explain the following metal forming processes with a neat sketch:

a.	Wire Drawing 		b.	Impact Extrusion 		c.	Stretch forming.

(OR)

23.	Discuss the principle of operation of gas welding equipment with a neat sketch. Also explain the types of oxy - acetylene flames.

[P.T.O]

24.	Draw and explain any two operations performed on the following machines:

a.	Lathe 		b.	Shaper 		c.	Milling Machine.

(OR)

25.	Discuss about the following:

a.	Any two types of cast iron.

b.	Classification of stainless steel based on its micro-structure.

c.	Nickel and its alloys.