# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: ADVANCED STRENGTH OF MATERIALS    				Time: 3 hours

# Subject Code: 12ME307		         		        				 	Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

- The stress state at a point is given by a matrix :

[σij]=11060060−8600055MPa

Determine :

i.	the principle stress 														(4)

ii.	the direction cosines of the principle stresses. 								(4)

iii.	the maximum shear stress.													(2)

- The displacement field of a thin beam is shown in the figure below. Considering the bending stress only is

U(x,y)=Py6EI( 6Lx -3x2 – vy2);

V(x,y)= −P6EI[3Lx2-x3+3vy2(L-x)]

Where   	P= applied force

I=area moment of inertia about bending axis

L= length of the beam

Determine:

i.	 the vertical deflection and slope at the centroidal axis y=0.					(5)

ii.	the entire stress field														(5)

2.	Determine the shear force and bending moment equations for the figure and plot the results.

(OR)

[P.T.O]

3.	Explain the following

a.	Tresca Theory. 															(8)

b.	Von Misses Theory. 														(12)

4.	Using the Castigliano’s theorem , determine the vertical deflection at point A of the cantilever beam shown below. The stiffness of beam is EIz =10x104 N-m2, P = 500N, L= 1m and b=0.2m

(OR)

5.	Determine the deflection and slope of point A using the virtual load method for the figure shown below. Consider the effects for bending only.

6.	A beam section shown in the figure is made of steel, Es=210 GPa and brass, Eb=105 GPa welded together. The beam 1m long is cantilevered with a force at free end of 100N. Determine the maximum tensile stress in steel, maximum compressive stress in brass and deflection of free end. Make the equivalent section out of brass.

(OR)

7.	A steel tube 1.5m long has the cross section shown in the figure. The tube is transmitting a torque of 200N-m. Determine the average shear stress in each wall. E=210GPa, v=0.29.

[P.T.O]

8.	The equilateral cross section with the sides of length ‘a’ is shown in the figure. Determine the shear stress distribution, if the section is transmitting a torque T

(OR)

9.	Determine the stress function for the stress field shown in the figure. Evaluate the stress field.