# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: FINITE ELEMENT ANALYSIS              					 Time: 3 hours

# Subject Code: 12ME308				         		   				 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Solve the following system of equation using Gauss elimination method:

X1 –X2 +X3 =1

3X1 +2X2 3X3 =6

2X1 –5X2 +4X3 =5

2.	The following differential equation is available for a physical phenomenon:

d2ydx2+100=0 ;  0 ≤ x ≤ 10.   The boundary conditions are: y(0) = 0 and y(10)=0.  By using weighted Residual approach, find an approximate solution of the above differential equation.

(OR)

3.	Derive the shape function for a 2D Rectangular element (Use local coordinate system having origin at the middle of the element).

4.	Find the stresses induced in the axially loaded stepped bar shown in the figure below. The bar has cross sectional areas A1 and A2 over the lengths L1 and L2 respectively. Assume the following data: A1= 2cm2 and A2= 1cm2, L1=L2=10cm, E1=E2=2x107N/cm2, P=100N.

2

10 cm

10 cm

1

3

1

2

P

(OR)

5. 	Find the natural frequencies of longitudinal vibrations of the unconstrained stepped bar shown in the figure below.

length(1)=L/2

length(2)=L/2

Q3

Q1

Q2

X

Element 1

A (1) =2A

Element 2

A (2) = A

1

2

6.	Evaluate the Stiffness matrix for a 2D triangular element relevant for solving 2D field problems.

(OR)

[P.T.O]

1cm

y

x

A

M

B

1cm

7.	Determine element shear stresses for a steel shaft of square cross section as shown in the figure below. Take G=8x106 N/cm2, Angle of twist=0.01o/cm.



8.	A wall of an industrial oven consists of three different materials. The first layer is composed of  5 cm of insulating cement with a clay binder that has a thermal conductivity of 0.08 W/m K. The second layer is made from 15 cm of asbestos board with a thermal conductivity of 0.074 W/m K. The exterior consists of 10 cm common brick with a thermal conductivity of 0.72 W/m K. The inside (inlet) wall temperature of the oven is 200°C, and the outside air is 30°C with a convection coefficient of 40 W/m2 K. Determine the temperature distribution along the composite wall.

(OR)

9. 	a. 	Define Boundary element method (BEM) and explain the basic steps in BEM.	(2+4)

b. 	Write the advantages and disadvantages of the Boundary Element Method relative to 	Finite Element Method in detail.											(4+4)

c. 	Enumerate the types of Boundary element methods.							(6)