Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ADVANCED THERMODYNAMICS AND HEAT TRANSFER   Time: 3 hours

Subject Code: 12ME311	         						                            Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	a.	One kg of water at 273 K is brought into contact with a heat reservoir at 373 K. When the 		water has reached 373 K, find the entropy change of the water, of the heat reservoir, and of 		the universe. 																	(4)

b.	If water is heated from 273 K to 373 K by first bringing it in contact with a reservoir at 323 K and then with a reservoir at 373 K, what will the entropy change of the universe be? 																				(4)

c.	Explain how water might be heated from 272 K to 373 K with almost no change in the 			entropy of the universe.														(4)

d.	A system has a heat capacity at constant volume Cv=AT2  where  A =0.042 J/K3. The system is originally at 200 K and a thermal reservoir at 100 K is available. What is the maximum amount of work that can be recovered as the system is cooled down to the temperature of the reservoir?													(8)

(OR)

2.	An air preheater  is used to cool the products of combustion from a furnace while heating the air to be used for combustion. The rate of flow of products is 12.5 kg/s and the products are cooled from 300oC to 200oC, and for the products at this temperature Cp= 1.09 kJ/kg K. The rate of air flow is 11.5 kg/s, the initial air temperature is 40oC, and for the air Cp=1.005 kJ/Kg K.

a.	Estimate the initial and final availability of the products.

b.	What is the irreversibility of the process?

c.	If the heat transfer from the products occurs reversibly through the heat engines, what is 	the final temperature of the air? What is the power developed by the heat engine? Take 	T0=300K and neglect pressure drop for both the fluids and heat transfer to the 	surroundings.

3.	a.	Derive the Virial’s equation of state.											(12)

b.	A mass of 0.25 kg of an ideal gas has a pressure of 300 kPa, a temperature of 80 oC and a volume of 0.07 m3. The gas undergoes an irreversible adiabatic process to a final pressure of 300 kPa and a final volume of 0.1 m3 , during which the work done on the gas is 25 kJ. Evaluate the cp and cv of the gas and the increase in entropy of the gas.				(8)

(OR)

4.	A mass of air is initially at 260oC and 700 kPa and occupies 0.028 m3. The air is expanded at constant pressure to 0.084 m3. A polytropic process with n = 1.50 is then carried out, followed by a constant temperature process which completes a cycle. All the processes are reversible.

a. 	Sketch the cycle in the p-v and t-s planes.		b. 	Find the heat received and heat rejected in the cycle. 		c. 	Find the efficiency of the cycle.

5.	A certain natural gas has the following volumetric analysis: 72% CH4, 9%H2, 14%N2, 2%O2, 3% CO2. This gas is now burned with the stoichiometric amount of air that enters the combustion chamber at 200C, 1 atm, and 80 % relative humidity. Assuming complete combustion and a total pressure of 1atm, determine the dew point temperature of the products.

[P.T.O]

(OR)

6.	Liquid octane (C8H18) enters the combustion chamber of a gas turbine steadily at 1 atm and 250C, and it is burned with air that enters the combustion chamber at the same state. Determine the adiabatic flame temperature for:

a.	Complete combustion with 100% theoretical air.

b.	Complete combustion with 400% theoretical air.

c.	Incomplete combustion (some CO in products) with 90 % theoretical air.

7.	A 1 shell-2 tube pass steam condenser consists of 3000 brass tubes of  20mm diameter .Cooling water enters the tubes at 200C with a mean flow rate of 3000 kg/s. The heat load of the condenser is 2.3x108W when the steam condenses at 500C.  Determine

a.	the outlet temperature of the cooling water.

b.	the overall heat transfer coefficient.

c.	the tube length per pass using NTU method.

d.	the rate of condensation of steam if  hfg = 2380kJ/kg.

(OR)

8.	Derive an expression for rate of condensation, on laminar film-wise condensation, on a vertical plate.

9.	Compulsory :

a.	Explain the various techniques adopted for electronic equipment cooling.			(10)

b.	Detail the heat transfer rate in treated surfaces and rough surfaces and also discuss the effect of friction factor on those surfaces.										(10)