# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: PRODUCTION AND OPERATIONS MANAGEMENT		 Time: 3 hours

# Subject Code: 12MS313							                 	 	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

A firm producing hand trolley has the following master production schedule (MPS).

Week 1 2 3 4 5 6 7 8

Demand 200 - - 240 - 240 - 220

Each hand trolley has a bin, two wheel assemblies, two handle bars and a mounting base. 	Each wheel assembly has a wheel and two bearings. Order Quantities, lead times and 	inventories on hand at the beginning of period 1 are shown below :

| Part | Order Quantity | Lead Time (Week) | Inventory on Hand |
| --- | --- | --- | --- |
| Hand Trolley | 350 | 1 | 220 |
| Bin | 400 | 2 | 250 |
| Wheel Assemblies | 800 | 2 | 120 |
| Handle Bars | 850 | 2 | 105 |
| Mounting Base | 250 | 1 | 250 |
| Wheel | 600 | 2 | 300 |
| Bearing | 1500 | 2 | 200 |

A shipment of 800 wheel assemblies is already scheduled to be received at the beginning of week 2. Also, another shipment of 250 mounting base is scheduled to be received at the beginning of week 3. Complete the material requirements plan for the Bin, Wheel assemblies, Handle Bars, Mounting base, Wheel and Bearing and show what quantities of orders must be released and when they must be released in order to satisfy the MPS.

2.	Explain in detail the various variables that affect labour productivity. Under what conditions should we expect employees whose needs are satisfied to be productive?

(OR)

3.	In a binding press, binding of large size books consists of the following operations:

a.	Stitching of the pages

b.	Trimming the book

c.	Cover making

d.	Joining the book with the cover and finishing.

The first operation is carried by Mr. Sam and the rest of the operations are performed by Mr. Victor. It is decided to use conventional time study technique for the first operation and work sampling method for the rest of the operations. The effective time available per shift is 480 minutes and the number of acceptable book bindings done during this period is 8. The details of the time study data of the first operation are shown in Table 1. The performance rating of Mr. Sam is 105 per cent. The details of the work sampling data are shown in Table 2.

Table 1. Time Study Data of Operation 1.

Cycle Time 18 19 20 21 22

Frequency 2 3 5 4 1

[P.T.O]

Table 2. Work Sampling Data

Operation Frequency of Performance Performance Rating

2 7 120%

3 21 125%

4 22 95%

Find the standard time of the entire job of books binding per book by assuming an allowance of 10 per cent to all the operations.

4.	A Company assembles and mails advertising packages for customers on a contract basis. One such contract has just been signed and the company is developing a layout for the assembly line. These tasks, their predecessor tasks, and task times have been identified and estimated, as shown below. The contract specifies that 50,000 mailers must be processed in five working days, the company works only one 8 hour shift per day, and employees are allowed two 15-miunte coffee breaks per shift.

a.	Draw the diagram of the precedence relationships.

b.	Compute the cycle time in minutes.

c.	Compute the minimum number of workstations for the contract.

d.	Combine the tasks into workstations to minimize idle time by using the incremental utilization heuristic. Evaluate your proposal.

| Task | Tasks that must Immediately Proceed | Task Time (Minutes/ Mailer) |
| --- | --- | --- |
| A. Inspect materials for quality | - | 0.30 |
| B. Prepare kits for assembly | A | 0.20 |
| C. Assemble and glue envelopes | B | 0.15 |
| D. Get and fold circular | B | 0.10 |
| E. Attach address to envelope | C | 0.10 |
| F. Place circular in envelope and seal envelope | D,E | 0.15 |
| G. Prepare plastic sleeve to fit over envelope | F | 0.05 |
| H. Seal package into plastic sleeve | G | 0.15 |
| I. Process addressed mailers through postage machine | H | 0.15 |
| J. Deliver mailers to bin | I | 0.05 |

(OR)

5.	The annual demand of a product is 48,000 units. The average lead time is 4 weeks. The standard deviation of demand during the average lead time is 75 units/week. The cost of ordering is ` 400 per order. The cost of purchase of the product per unit is `10. The cost of carrying per unit per year is 15 per cent of the purchase price. The maximum delay in lead time is 2 weeks and the probability of this delay is 0.25. Assume a service level of 0.95.

a.	If Q system is followed, find the reorder level.

b.	If P system is followed, find the maximum inventory level.

6.	Discuss the steps in constructing  X and R charts.

(OR)

7.	The following data were obtained over a five-day period to indicate X and R control chart for a quality characteristic of a certain manufacturing product that had required a substantial amount of rework. All the figures apply to the product made on a single machine by a single operator. The sample size was 5. Two samples were taken per day. Comment on the process using Xand R charts.

[P.T.O]

| Sample Number | Observations |
| --- | --- |
|  | 1 2 3 4 5 |
| 1 2 3 4 5 6 7 8 9 10 | 10 12 13 8 9 7 10 8 11 9 11 12 9 12 10 10 9 8 13 11 8 11 11 7 7 11 8 8 11 10 10 12 13 13 9 10 12 12 10 12 12 13 11 12 10 10 13 7 9 12 |

8.	Name the ten decision areas of Operations Management and the issues concerning each one of them.

(OR)

9.	Define production system. How does the concept of a production system help in the understanding of POM?