Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DIGITAL SIGNAL PROCESSING AND MEDIA APPLICATIONS					      																       Time: 3 hours

Subject Code: 12MT213						       		    		              Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Find the linear convolution between x(n) = {5} and h(n) = {7}.

2. 	Write the formula to find the DTFT of a sequence x(n).

3. 	State any one difference between chebyshev filter and elliptic filter.

4. 	Define cut-off frequency of a filter.

5. 	Write the causal Blackman window formula.

6. 	What is the condition for linear-phase characteristics of an FIR filter?

7. 	What are the three forms of fixed point representation?

8. 	What are the two types of limit cycles due to round-off effects in a digital filter?

9. 	MAC stands for _______________.

10. 	Mention the addressing modes in digital signal processors.

PART – B  (5 x 3 = 15 MARKS)

11. 	State the advantages of digital signal processing over analog signal processing.

12. 	Write the difference between butterworth and chebyshev filter.

13. 	Define Gibb’s phenomenon.

14. 	Define dead band and product quantization error.

15. 	Write short notes on musical sound processing.

PART – C  (5 x 15 = 75 MARKS)

16. 	Check the following systems for linearity, time invariance, causality, static, stability.

a.	y(n) = 2x(n)

b.	y(n) = x(–n)

c.	y(n) = x2(n)

(OR)

17. 	A sequence is defined by

x(n) = n+1	0 ≤ n ≤ 7

Find the DFT of the sequence x(n) by DIF – FFT algorithm.

18. 	Design a digital Chebyshev filter to satisfy the constraints.

Apply impulse invariant method with T = 1 sec.

(OR)

19. a.	Develop cascade and parallel form realization of the system described by the difference 	equation.

(10)

[P.T.O]

b.	Determine the direct form II realization for the following system.

(5)

20. 	Design an FIR lowpass filter of length N=7 to meet the following specifications using Hamming window.

Also determine the frequency response of the FIR filter.

(OR)

21.	Determine the coefficients of a linear phase FIR filter of length N=15 that has a symmetric unit sample response. The frequency response of the FIR filter is given by its samples that satisfy the conditions.

22.	Consider an IIR filter y(n) = ay(n–1)+x(n) with input . Find the limit cycle effect and dead band for 	a.	   	b.	 . Assume that the register length is 4 bits (including sign bit).

(OR)

23.	Consider a second order IIR filter with system function   Determine the effect of quantization on pole locations of the given system function in direct form and cascade form with b=5 bits (including sign bit).

24. 	Explain the following in detail with a neat diagram:

a.	Pipelining.																	(8)

b.	Harvard architecture.															(7)

(OR)

25.	a.	Why is cache memory required in a digital signal processor?		  				(5)

b.	Explain any one application of digital signal processing with a neat diagram. 		(10)