Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MECHANICS OF SOLIDS							           	       Time: 3 hours

Subject Code: CE203						   		        		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is factor of safety?

2.	Define Poisson’s ratio.

3.	Bending moment is __________ where shear force is __________.

4.	Write down the relationship between shear force and bending moment.

5.	What is the simple bending equation?

6.	Name the types of springs.

7.	On principal planes __________ is zero.

8.	Analytical and graphical methods are used for finding the stresses on an __________ section.

9.	Where __________ is maximum, __________ is zero.

10.	Long column fails by __________.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the three elastic constants? Write down the relationship between them.

12.	Draw SFD and BMD for the simply supported beam with an eccentric point load.

13.	A solid cylindrical shaft is to transmit 300 kW power at 100 rpm.  If the shear stress is not to exceed 80 N/mm2, find its diameter.

14.	The tensile stresses at a point across two mutually perpendicular planes are 120 N/mm2 and 60 N/mm2.  Determine the normal, tangential and resultant stresses on a plane inclined at 30° to the axis of the minor stress.

15.	A cantilever of length 3 m carries two point loads of 2 kN at the free end and 4 kN at a distance of 1 m from the free end.  And the deflection at the free end.  Take E=2x105 N/mm2 and I = 108 mm4.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	A brass bar having cross sectional area of 1000 mm2 is subjected to axial forces as shown 	in figure below:

Find the total elongation of the bar. Take E = 1.05x105 N/mm2					(8)

[P.T.O]

b.	Calculate the modulus of rigidity and bulk modulus of a cylindrical bar of diameter 30 mm 	and of length 1.5 m if the longitudinal strain in a bar during a tensile stress is four times the 	lateral strain.  Find the change in volume when the bar is subjected to a hydro static 	pressure of 100 N/mm2.  Take E=1x105 N/mm2.									(7)

(OR)

17.	A closed cylindrical vessel made of steel plates 4 mm thick with plane ends, carries fluid under a pressure of 3 N/mm2.  The diameter of cylinder is 25 cm and length is 75 cm. Calculate the longitudinal and hoop stresses in the cylinder wall and determine the change in diameter, length and volume of the cylinder.  Take E=2.1x105 N/mm2 and μ=0.286.

18.	A cantilever 1.5 m long is loaded with a uniformly distributed load of 2 kN/m over a length of 1.25 m from the free end.  It also carries a point load of 3 kN at a distance of 0.25 m from the free end.  Draw the shear force and bending moment diagrams of the cantilever.

(OR)

19.	Draw the shear force and bending moment diagrams for the over hanging beam carrying uniformly distributed load of 2 kN/m over the entire length as shown in figure below. Also locate the point of contraflexure.

20.	A cast iron beam is of T-section as shown in figure below.  The beam is simply supported on a span of 8 m.  The beam carries a udl of 1.5 kN/m on the entire span.  Determine the maximum tensile and maximum compressive stress.

(OR)

21.	A close coiled helical spring of 10 cm mean diameter is made up of 1 cm diameter rod and has 20 turns.  The spring carries an axial load of 200 N.  Determine the shearing stress. Taking the value of modulus of rigidity as 8.4 x 104 N/mm2, determine the deflection when carrying this load.  Also calculate the stiffness of the spring and the frequency of free vibration for a mass hanging from it.

[P.T.O]

22.	The normal stress in two mutually perpendicular directions are 600 N/mm2 and 300 N/mm2 both tensile.  The complimentary shear stresses in these directions are of intensity 450 N/mm2.  Find the normal and tangential stresses on the two planes which are equally inclined to the planes carrying the normal stresses mentioned above.

(OR)

23.	The tensile stresses at a point across two mutually perpendicular planes are 120 N/mm2 and 60 N/mm2.  Determine the normal, tangential and resultant stresses on a plane inclined at 30° to the axis of the minor stresses.  Check the values by Mohr’s circle method.

24.	An overhanging beam ABC is loaded as shown in figure below.  Find the slopes over each support and at right end.  Find also the maximum upward deflection between the supports and the deflection at the right end.

Taka E= 2x105 N/mm2 and I=5x108 mm4.

(OR)

25.	a.	Derive the expression for crippling load when both ends of the column are hinged.	(8)

b.	A 1.5 m long column has a circular cross section of 5 cm diameter.  One of the ends of the column is fixed in direction and position and other end is free. Taking factor of safety as ‘3’  calculate the safe load using Euler’s formula.  Take E = 1.2x105N/mm2.			(7)