Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ENGINEERING MECHANICS							       	       Time: 3 hours

Subject Code: CE204							       		    		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Engineering Mechanics.

2.	What is a co-planar force?

3.	Name the different types of supports of a beam.

4.	What is the use of a Wedge?

5.	What is centre of gravity?

6.	What is radius of gyration?

7.	Name the systems followed for locating a particle along a curved path.

8.	What is Newton’s second law of motion?

9.	Write down the formula for kinetic energy.

10.	Define efficiency.

PART – B  (5 x 3 = 15 MARKS)

11.	Define parallelogram law of forces.

12.	a.	What are the types of frictional forces?

b.	Write down the application of dry friction.

13.	Define parallel axis theorem.

14.	A motorist travelling at speed of 18 kmph suddenly applies brakes and comes to rest after skidding 75 m.  Find the time required for the bike to stop.

15.	A cricket ball hit at a height of 1.5 m from the ground by a batsman with a velocity of 20 m/sec, at an angle of 30° to the horizontal was caught by the fieldman at a height of 50 cm from the ground.  Find out the distance between the two players.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	If two forces F1= 20 kN and F2= 15 kN act on a particle as shown in figure below find 	their resultant by 		i.	Parallelogram law 	ii.	Triangle law.					(10)

b.	The greatest and least resultant of two forces acting on a particle are 35 kN and 5 kN 	respectively.  If 25 kN is the magnitude resultant for the given system of forces, prove 	that the forces are at right angles.												(5)

(OR)

[P.T.O]

17.	a.	A lamp weighing 5 N is suspended from the ceiling by a chain.  It is pulled aside by a 	horizontal chord until the chain makes an angle of 60° with the ceiling as shown in figure 	below.  Find the tensions in the chain and the chord by applying Lame’s theorem.	(8)

b.	Two forces P and Q are acting at a point O as shown in figure below. The resultant force 	is 400 N and angles β and  are 35° and 25° respectively.  Find the two forces P and Q.																					(7)

18.	A system of parallel forces acting on a rigid bar ABCD are shown in figure below. Reduce this system to:

a.	single force and couple at ‘A’.

b.	single force and couple at ‘C’.

c.	single force and couple at ‘D’.

d.	A single force.

(OR)

19.	Determine the resultant of the system of forces acting on the channel section shown in figure below.  Taking M= 10 Nm, find the moment of the couple, if the resultant of the force system is to pass through point ‘A’ and ‘B’ and ‘C’.

[P.T.O]

20.	Find the forces in the members AB, AC and BC of the truss shown in figure below using method of joints.

(OR)

21.	Determine the polar moment of inertia of the I section shown in figure below. All dimensions are in mm.

22.	The motion of a particle is given by the relation S=3t3−12t2+7.5t+22.5where ‘S’ is expressed in meters and ‘t’ in seconds.  Determine the position, velocity and acceleration when t = 4sec.  Also find the net displacement of the particle during the interval from t = 1 sec to t = 4 sec.

(OR)

[P.T.O]

23.	The rotation of a 1 m rod, OA shown in figure below is defined by θ=0.15 t2 where θ is expressed in radians and ‘t’ in sec. A block ‘B’ on the rod OA, slides along the rod in such a way that its radial distance from ‘O’ is r = 1–0.15t2 where ‘r’ is expressed in meters and ‘t’ in sec.  Determine the resultant velocity and resultant acceleration of block ‘B’ when the rod has rotated through 45°.

24.	An aeroplane of mass 8t is flying at a rate of 250 kmph, at a height of 2 km above the ground level.  Calculate the total energy possessed by the aeroplane.

(OR)

25.	a.	A mass of 50 kg has an initial velocity of 15 m/sec horizontally on smooth surface. 	Determine the value of the horizontal force that will bring the mass to rest in 4 sec.	(8)

b.	A food packet having mass of 10 kg is dropped with initial velocity of 5 m / sec from very 	high altitude.  If the friction resistance due to atmosphere is 80 N, determine the velocity of 	the food packet when it has fallen at 10 m height.									(7)