Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: FLUID MECHANICS AND MACHINERY				      	       Time: 3 hours

Subject Code: CE212								       			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Specific Volume.

2.	What do you mean Vacuum pressure?

3.	Write the Bernoulli's equation.

4.	Differentiate Laminar and Turbulent flow.

5.	Define Co-efficient of Discharge.

6.	What are the Minor losses in the pipe?

7.	What is the force exerted by a jet of water on a stationary Vertical plate?

8.	Define Specific speed of a pump.

9.	State the Momentum equation.

10.	Define Unit Power.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Surface tension.

12.	What is meant by vortex flow?

13.	What is meant by a syphon? Where is it used?

14.	What is an air vessel? Describe the function of it.

15.	How will you classify the turbines?

PART – C  (5 x 15 = 75 MARKS)

16.	The dynamic viscosity of oil used for lubrication between a shaft and a sleeve is 6 poise. The shaft is of diameter 0.4 m and rotates at 190 rpm. Calculate the power lost in the bearing for a sleeve length of 90 mm.The thickness of the oil film is 1.5 mm.

(OR)

17.	Distinguish between manometer and mechanical gauges. What are the different types of mechanical pressure gauges?

18.	The velocity vector in a fluid is given by:

v=4x3i-10x2yj+2tk
Find the velocity and acceleration of a fluid particle at (2, 1, 3) at time t=1.

(OR)

19.	Water is flowing through a pipe having diameter 300 mm and 200 mm at the bottom and upper ends respectively. The intensity of pressure at the bottom end is 24.525 N/cm2 and the pressure at the upper end is 9.81N/cm2. Determine the difference in the datum head if the rate of flow through the pipe is 40 lit/s.

20.	What is a venturimeter? Derive the expression of the discharge through a venturimeter.

(OR)

21.	An oil of specific gravity 0.9 and viscosity 0.06 poise is flowing through a pipe of diameter 200mm @ 16 lit/s. Find the head loss due to friction for 500 mm length of pipe? Find Power required to maintain the flow.

[P.T.O]

22.	A nozzle of 50mm diameter delivers a stream of water at 20m/s perpendicular to a plate that moves away from the jet at 5m/s. Find:

a.	The force on the plate.

b.	The work done.

c.	The efficiency of the jet.

(OR)

23.	Explain the following terms as they are applied to a centrifugal pump:

a.	Static suction lift.

b.	Static suction head.

c.	Static discharge head.

d.	Total static head.

24.	A Pelton wheel develops 8000 KW under a net head of 130 m at a speed of 200 rpm. Assuming the co-efficient of velocity for the nozzle 0.98, hydraulic efficiency is 87%, speed ratio is 0.46 and jet diameter to wheel diameter ratio is 1/9, Determine:

a.	the discharge required.

b.	The diameter of the wheel.

c.	The diameter and the number of jets required.

d.	The specific speed.

(Mechanical Efficiency =75%).

(OR)

25.	Explain the principle and working of Gear Oil pump.