Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MECHANICS OF DEFORMABLE BODIES – I	                   	       Time: 3 hours

Subject Code: CE240						       		        		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Poisson’s ratio = (________strain / _______strain).

2.	If E is Young’s modulus, K is bulk modulus and 1/m is Poisson’s ratio then:

E = _______K (1-2/m).

3.	Value of shear stress in principal plane is _________.

4.	The circumferential stress in a thin cylinder is also known as ______ stress.

5.	Bending moment is maximum where shearing force is zero or _______.

6.	What is a continuous beam?

7.	Shear stress in a section will be maximum at _________.

8.	The curvature of the axis of beam in bending is directly proportional to _________ and inversely proportional to__________.

9.	Torque is equal to product of force (tangential) applied and ________of the shaft.

10.	When a circular shaft is subjected to torsion, ________ stress are setup in the material of the shaft.

PART – B  (5 x 3 = 15 MARKS)

11.	State Hooke’s law.

12.	Write the expression for the following in terms of principal stresses.

a.	Normal stress.

b.	Tangential stress in a plane at θ.

c.	Maximum shear stress.

13.	A cantilever beam of 4m length carries a point load of 1kN for every 1m interval. Draw the bending moment diagram and shear force diagram.

14.	State the assumptions in theory of simple bending.

15.	Write the expression for principal stress and maximum shear stress when a shaft is subjected to bending and torsion.

PART – C  (5 x 15 = 75 MARKS)

16.	A steel rod of 20 mm diameter passes through a copper tube of 50 mm external diameter and  40 mm internal diameter. The tube is closed at each end by rigid plates of negligible thickness. The nuts are tightened lightly home on the projecting parts of the rod. If the temperature of the assembly is raised by 50°C, calculate the stresses developed in copper and steel. Take E for steel and copper as 200 GN/m2 and 100 GN/m2 and α for steel and copper as 12 x 10-6 per °C and 18 x 10-6 per °C.

(OR)

17.	A steel rod 20 mm in diameter passes centrally through a steel tube of 25 mm internal diameter and 30 mm external diameter. The tube is 800 mm long and is closed by rigid washers of negligible thickness which are fastened by nuts threaded on the rod. The nuts are tightened until the compressive load on the tube is 20kN. Calculate the stresses in the tube and the rod.

Find the increase in these stresses when one nut is tightened by one-quarter of a turn relative to the other. There are 4 threads per 10 mm. Take E = 2 x 105 N/mm2.

[P.T.O]

18.	A cylindrical thin drum 80 cm in diameter and 3 m long has a shell thickness of 1 cm. If the drum is subjected to an internal pressure of 2.5 N/mm2, determine:

a.	Change in diameter.															(5)

b.	Change in length.																(5)

c.	Change in volume.															(5)

Take E = 2 x 105 N/mm2; Poisson’s ratio = 0.25

(OR)

19.	At a certain point in a strained material, the intensities of stresses on two planes at right angles to each other are 20 N/mm2 and 10 N/mm2 both tensile. They are accompanied by a shear stress of magnitude 10 N/mm2. Find graphically or otherwise, the location of principal planes and evaluate the principal stresses.

20.	A ladder 6 m long weighing 25 kg/meter run, rests against a smooth vertical wall with the bottom resting on rough ground, the vertical distance between the ends of the ladder being 4 m. A man weighing 65 kg is standing on the ladder at 1.5 meter from the bottom end and another man weighing 70 kg on the ladder at 3 meter from the bottom. Construct the axial thrust, shear force and BM diagrams for the ladder and calculate the reactions at the ground and at the wall.

(OR)

21.	A horizontal beam, 30 m long, carries a uniformly distributed load of 10 kN/m over the whole length and a concentrated load of 30 kN at the right end. If the beam is freely supported at the left end, find the position of the second support so that t he bending moment on the beam should be as small as possible. Draw the diagrams of shearing force and bending moment and indicate the principle values.

22.	A water main of 500 mm internal diameter and 20 mm thick is running full. The water main is of cast iron and is supported at two points 10 m apart. Find the maximum stress in the metal. The cast iron and water weigh 72,000 N/m3 and 10,000 N/m3 respectively.

(OR)

23.	a.	A leaf spring carries a central load of 3000 N. The leaf spring is to be made of 10 steel 	plates 5 cm wide and 6 mm thick. If the bending stress is limited to 150 N/mm2 determine:

i.	Length of the spring														(5)

ii.	Deflection at the centre of the spring										(5)

Take E = 2 x 105 N/mm2.

b.	A laminated spring 1 m long is made up of plates each 5 cm wide and 1 cm thick. If the 	bending stress in the plate is limited to 100N/mm2, how many plates would be required to 	enable the spring to carry a central point load of 2 kN? If E = 2.1 x 105 N/mm2, what is the 	deflection under the load?														(5)

24.	A hollow circular shaft 20 mm thick transmits 300 kW power at 200 r.p.m. Determine the external diameter of the shaft if the shear strain due to torsion is not to exceed 0.00086. Take modulus of rigidity = 0.8 x 105 N/mm2.

(OR)

25.	A closely coiled helical spring of round steel wire 10 mm in diameter having 10 complete turns with a mean diameter of 12 cm is subjected to an axial load of 200N. Determine:

a.	The deflection of the spring													(5)

b.	Maximum shear stress in wire													(5)

c.	Stiffness of the spring.														(5)

Take modulus of rigidity,C = 8 x 104 N/mm2.