Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MECHANICS OF DEFORMABLE BODIES – II			       Time: 3 hours

Subject Code: CE241			       					               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Macaulay’s method is more suitable for beams with _______ loads.

2.	Write the factors affecting deflection of a beam.

3.	What do you mean by slenderness ratio?

4.	How does the strut fail?

5.	How does the hoop stress vary across the thickness of a thick cylinder?

6.	_______ theory is more suitable for brittle materials

7.	Where does the shear centre lie for a beam having two axes of symmetry?

8.	Write down the expression for Winkler Bach formula.

9.	Under unsymmetrical bending the resultant deflection of a beam is _______ to the neutral axis.

10.	What is the product of inertia of a section about its principal axis?

PART – B  (5 x 3 = 15 MARKS)

11.	What do you mean by Conjugate beam?

12.	Differentiate long, medium and short columns.

13.	What is shrink fit of a cylinder?

14.	What is shear centre and shear flow?

15.	What are the reasons for unsymmetrical bending?

PART – C  (5 x 15 = 75 MARKS)

16.	A Cantilever beam 2.5 m long is loaded with a UDL of 10 KN/m run over a length of 1.5 m from the fixed end. Determine the slope and deflection at the free end of the cantilever. Take M.I.=9500X104 mm4, E=2X105 N/mm2.

(OR)

17.	A beam AB of span 8m is loaded with a point load of 20 KN at its centre. Moment of inertia for the middle 4m is 1.2 I and for the remaining portions I. Find the midspan deflection of beam using conjugate beam method. Take E=2X105 N/mm2 , I=2X108 mm4.

18.	Find the inside diameter of a cast iron column of 200 mm outside diameter which is 5 m long and has to support a safe axial load of 600 kN, one end being rigidly fixed. Use a factor of safety of 5 in conjunction with Rankine’s formula. a = 1/1600, σc=567 MN/m2.

(OR)

19.	Derive the expression for Euler’s crippling load of a column with one end hinged and the other end fixed.

20.	A pipe of 200 mm internal diameter and 50mm thickness carries a fluid at a pressure of 5 MN/m2. Calculate the maximum and minimum intensities of circumferential stress across the section.

(OR)

[P.T.O]

21.	Determine the diameter of a bolt which is subjected to an axial pull of 9 kN together with a transverse shear force of 4.5 kN using	  a.	Maximum Principal stress theory	b.	Maximum Principal strain theory. Given the elastic limit in tension = 225 N/mm2, Factor of safety = 3 and Poisson’s ratio = 0.3.

22.	Derive the expression to find shear centre for channel section.

(OR)

23.	At the critical section of a crane hook, trapezium in section, the inner and outer sides are 40 mm and 25 mm respectively and depth is 75 mm. The centre of curvature of the section is at a distance of 60 mm from the inner fibers and the load line is 50 mm from the inner fibers. If the maximum stress is not to exceed 120 MN/m2, what maximum load can the hook carry?

24.	A beam of T section (Flange: 100 mm x 20 mm, Web: 150 mm x 10 mm) is 2.5 m in length and is simply supported at its ends. It carries a load of 3.2 kN inclined at 20˚ to the vertical and passing through the centroid of section. If E = 200 GN/m2, calculate maximum tensile stress and maximum compressive stress.

(OR)

25.	A cantilever of rectangular section 40 mm x 60 mm is subjected to an inclined load P at the free end. The inclination at the load is 25˚ to the vertical. If the length of cantilever is 2 m and maximum stress due to bending is not to exceed 800 MN/m2, determine the value of P.