Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ENGINEERING MECHANICS							      	       Time: 3 hours

Subject Code: CE242			       				        		       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Distinguish between unit and dimension.

2.	State Lami’s theorem.

3.	What are the equations of equilibrium of a rigid body?

4.	Frictional force is _____________ of the area of contact.

5.	Differentiate between centroid and center of gravity.

6.	Define radius of gyration.

7.	What do you mean by instantaneous velocity?

8.	A curve defined by the tip of the velocity vector in the curvilinear motion of particle is 	_______.

9.	Define work done by a force.

10.	State the principle of impulse and momentum.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the classifications of a force system?

12.	Explain the different types of supports.

13.	The force required to pull a body of weight 50 N on a rough horizontal plane is 15 N.  Determine the co-efficient of friction if the force is applied at an angle of 150 with the horizontal.

14.	A body is moving with a velocity of 2 m/s. After 4 seconds the velocity of the body becomes   5 m/s. Find the acceleration of the body.

15.	A ball is dropped on a horizontal floor from which it rebounds to a height of 16 m. If the co-efficient of restitution between the floor and the ball is 0.8 find the height from which the ball was dropped.

PART – C  (5 x 15 = 75 MARKS)

16. 	The sum of two concurrent forces P and Q is 270 N and their resultant is 180 N. The angle between the force P and resultant R is 900. Find the magnitude of each force and angle between them.

(OR)

17.	Forces 32 kN, 24 kN, 24 kN and 120 kN are concurrent at the origin and are respectively directed through the points whose coordinates are A(2,1,6), B(4,-2,5), C(-3,-2,1) and D(5,1,-2). Determine the resultant of the system.

[P.T.O]

18.	A beam AB of span 8 m, overhanging on both sides, is loaded as shown in the figure below.  Calculate the reactions at both ends.

(OR)

19.	A ladder 5 m long, and of 250 N weight is placed against a vertical wall in a position where its inclination to the vertical is 300. A man weighing 800 N climbs the ladder. At what position will he induce slipping? The co-efficient of friction for both the contact surfaces of the ladder i.e., with the wall and the floor is 0.2.

20.	Find the forces in the members AB and AC of the truss shown in the figure below using method of section.

(OR)

21.	Calculate the moment of inertia of the section as shown in figure about the XX and YY axis 	through the centriod.

22.	A shot is fired with a bullet with an initial velocity of 40 m/s from a point 20 m in front of a vertical wall 10 m high.  Find the angle of projection with horizontal to enable the shot to just clear the wall.

(OR)

23.	A particle moves along x-axis and its position is expressed as x = 3.5 t3 - 7 t2 where ‘x’ is in metres and ‘t’ is in seconds.  Determine: 	a. 	the position at t = 0, 1, 3, 7 seconds

b.	the displacement during t = 3sec, to t = 7 sec. 	c.	Find the average velocity during t = 3sec to t = 7 sec and instantaneous velocity at t = 3sec and t = 7 sec. 	d. Find the average acceleration during t = 3sec to t = 7 sec and instantaneous acceleration at t = 3sec and t = 7 sec.

[P.T.O]

24.	A mass 12 kg travelling to the right with a speed of 7.5 m/sec collides with another mass 24 kg, travelling to the left with a speed of 2 m/sec. If the coefficient of restitution is 0.6, find the velocities of the particles after collision and loss in kinetic energy. What is the impulse acting on either particle during the impact?

(OR)

25.	A pile hammer weighing 28 kN drops from a height of 800 mm on a pile of 15 kN. The pile penetrates 120 mm per blow. Assuming the motion of pile to be resisted by constant force, find the resistance to penetration of ground.