Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MECHANICS OF SOILS								      	       Time: 3 hours

Subject Code: CE247							       		    		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Water content.

2.	What is meant by consistency of soils?

3.	What is meant by soil water?

4.	Define Darcy’s law.

5.	Define coefficient of consolidation.

6.	Define compression Index.

7.	What is meant by stress isobar?

8.	What is meant by contact pressure?

9.	Name the various field tests to determine  shear strength.

10.	Define infinite slopes.

PART – B  (5 x 3 = 15 MARKS)

11.	What are different types of soil structures in nature?

12.	What are the various factors that affect the permeability of a soil stratum?

13.	Differentiate between primary consolidation and secondary consolidation.

14.	Write short notes on New Marks’ influence chart.

15.	What are different types of slope failures?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	What are uses of consistency limits? 			                              				(5)

b.	The minimum and maximum dry density of sand was found to be 1.50 and 1.70 gm/ml. Calculate the dry density corresponding to relative densities of 50% and 70%.		(10)

(OR)

17.	a.	What is the use of classification of soils? Discuss the IS Classification system.		(5)

b.	A sample of saturated soil has a water content of 35%. The specific gravity of solids is 2.65. Determine its void ratio, porosity, saturated unit weight and dry unit weight.  	(10)

18.	a.	What are the principles of a Flow net?  											(5)

b.	Calculate the coefficient of permeability of a soil sample, 6 cm in height and 50cm2 in cross sectional area, if a quantity of water equal to 430 ml passed down in 10 minutes, under an effective constant head of 40 cm. If oven dry weight of the specimen is 498 gm and has a specific gravity of 2.65 calculate the seepage velocity of water during the test.												          							(10)

(OR)

19.	a.	Explain falling head method of determination of co-efficient of permeability. 		(10)

b.	How would you find the flow net for a homogeneous earth dam without any filter? 	(5)

20.	a.	Differentiate between primary consolidation and secondary consolidation.      		(5)

b.	Describe the proctor compaction test and give its usage for construction of earth 					embankments.		           													(10)

[P.T.O]

(OR)

21.	a.	Write short notes on Method of compaction. 										 (5)

b.	There is a layer of soft clay 4m thick under a newly constructed building. The overburden pressure over the center of clay layer is 300 kN/m2. Compute the settlement if there is an increase in pressure due to construction of 100 kN/m2. Take 	Cc=0.50, G = 2.70. the water content of the deposit was found to be 50%.	         						(10)

22. 	a.	Explain stress distribution in soils for point loads by Boussinesq’s  equation.  		(10)

b.	Define allowable soil pressure.                                                 		      				(5)

(OR)

23.	a.	Differentiate between ‘total settlement’ and ‘differential settlement’. 				(8)

b.	A circular footing of 1.5m radius transmits a uniform pressure of 90 kN/ m2. Calculate   
          	the vertical stress at point 1.5m directly beneath its center	. 						(7)

24. 	Explain the principle of direct shear test.  What are the advantages?

(OR)

25. 	a.	Discuss the various methods for improving the stability of slope.  					(7)

b.	The stresses at failure on the failure plane in a cohesionless soil mass were                    shear stress = 4 kN/m2, and normal stress = 10kN/m2. Determine the resultant stress and the angle of internal friction of the soil.                                                                		(8)