Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: REINFORCED CONCRETE STRUCTURES - I		         	       Time: 3 hours

Subject Code: CE249				       						              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is the permissible compressive Stress for M20 concrete in bending?

2.	Modulus of elasticity of concrete is _________.

3.	If the ratio of long span to short span is less than two, the slab should be designed as _________.

4.	Minimum reinforcement in either direction in slabs shall not be less than _________.

5.	What is the slenderness ratio value for a long column?

6.	What is the minimum thickness that should be provided for a foundation?

7.	Write the formula for development length of a bar in a beam.

8.	Write the formula to calculate the shear in L beams.

9.	Minimum number of longitudinal bars provided in a rectangular column is _________.

10.	What is the maximum bending moment of a simply supported slab loaded with UDL?

PART – B (5 x 3 = 15 MARKS)

11.	Calculate the moment of resistance of a reinforced concrete beam of 300mm width and 500mm effective depth, which uses M-20 grade concrete.

12.	What are the forms of shear reinforcement in beams?

13.	When is combined rectangular footing provided? Also list the names of two shapes.

14.	Find the modular ratio of M20 concrete.

15.	Differentiate one way slab and two way slab.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Draw the stress-strain diagram for concrete and steel. 								(5)

b.	Write detailed notes on under, balanced and over reinforced section. 				(10)

(OR)

17.	Discuss in detail about working stress, limit state and ultimate load method of design.

18.	Design a continuous reinforced concrete beam of rectangular section to support a dead load of 10 kN/m and a service load of 15kN/m over three simply supported spans of 8 m each. Adopt M-20 concrete and Fe-415 HYSD bars.

(OR)

19.	Design a one way slab with a clear span of 3.5m, simply supported on 200mm thick concrete masonry walls to support a live load of 4 kN/m2. Adopt M-20 grade concrete and Fe-415 HYSD bars.

20.	Design the reinforcement in a circular column of diameter 400mm to support a factored load of 800kN together with a factored moment of 80KN.m. Adopt M-20 concrete and Fe-415 HYSD bars.

(OR)

[P.T.O]

21.	Design a reinforced concrete footing for a rectangular column of section 300mm by 500mm supporting an axial factored load of 1500kN. The safe bearing capacity of soil at site is 185 kN/m2. Adopt M-20 concrete and Fe-415 HYSD bars.

22.	The T beam section as shown in the figure below simply supported over a span of 10m, carries a load of 40,000N/m including the self-weight of the beam. Calculate the area of steel in tension and compression in the section. . Adopt M-20 concrete and Fe-415 HYSD bars.

(OR)

23.	Explain briefly about the design procedure of L beam.

24.	Design a two way slab for a room having clear dimesions 4000mmx5000mm. Take the load as 2000 N/m2 and floor finishes as 500 N/m2. Adopt M20 concrete and Fe415 HYSD bars.

(OR)

25.	Design a short column of reinforced concrete to carry an axial load of 11,00,000 N. Use M 20 concrete and σsc=190N/mm2.