Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: REINFORCED CONCRETE STRUCTURES - II			      Time: 3 hours

Subject Code: CE250			       						              	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is the normal range of rise values of steps of a staircase in a residential building?

2.	State the different types of retaining wall.

3.	Which type of water tank will be preferred for storing a large quantity of water?

4.	What is the minimum grade of concrete to be used for tanks having capacity more than 50 m3 as per IS 3370 (Part 2):2009.

5.	Mention the name of IRC code adopted for standard live loads in the design of bridges.

6.	State the minimum grade of concrete used in RCC bridges as per IRC: 21- 2000.

7.	Mention the name of IS code for provisions of earth quake effects.

8.	Mention the name of IS code for provisions of ductility in buildings.

9.	Give an advantage of strip method of analysis.

10.	Write the other names of methods of yield line analysis.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the assumptions made in Rankine’s theory pertaining to the design of retaining walls?

12. 	Differentiate underground water tanks from tanks resting on ground.

13. 	List out various types of reinforced concrete bridges and the factors influencing the selection of the type of a bridge.

14. 	Distinguish between portal method and cantilever method.

15. 	What is meant by yield line and mention the methods of yield line analysis.

PART – C  (5 x 15 = 75 MARKS)

16. 	Design a waist slab type dog Legged staircase of an office building using the following data:

Height between floors = 3.2 m.

Tread = 270 mm and riser = 160 mm.

Width of flight = landing width = 1.25 m.

The stairs are supported on 300 mm load bearing masonry walls at the outer edges of the landing, parallel to the risers.

Materials: M-20 grade concrete and Fe 415 HYSD Bars.

(OR)

17. 	Determine the stability and design the heel slab of a cantilever retaining wall for a road for the following details:

Height of wall from the bottom of base to top of stem = 6 m.

Superimposed load due to road traffic = 18 kN/m2

Unit weight of fill = 18 kN/m3

Angle of internal friction for fill material = 30°.

SBC of soil = 160 kN/m2

Coefficient of friction between concrete and ground = 0.4.

Use M 20 grade concrete and Fe4 15 steel.

[P.T.O]

18. 	Design an underground rectangular tank 4m x 6m x 2m deep. The subsoil consists of dune sand having a unit weight of 16 kN/m3 and angle of friction of 34°. The subsoil water level is at a great depth. Use M 25 concrete and Fe 415 steel.

(OR)

19. 	Design a circular water tank resting on ground for a capacity of 400,000 liters. The depth of water is to be 4 m, including a free board of 20 cm. Use M 25 concrete and Fe 415 steel.

20. 	Design a reinforced concrete slab culvert for a National Highway crossing to suit the   following data:

Carriage way – Two Lane (7.5 m wide).

Width of kerb = 600 mm.

Foot path 1m on either side.

Clear span = 6 m.

Wearing coat = 80 mm.

Width of bearing = 400 mm.

Materials: M 25 Grade concrete and Fe 415 grade HYSD Bars.

Loading – IRC class AA Tracked Vehicle.

Design the reinforced concrete slab deck and sketch the details of reinforcements in the longitudinal and cross section of the slab. The design should confirm to the specifications of IRC 6-2000 and IRC 21-2000 Codes.

(OR)

21.	The reinforced concrete slab panel of a reinforced concrete Tee beam and slab deck is 2 m wide between Tee beams and 4 m long between cross girders. Design the R.C. slab panel for IRC Class A loading using M-30 grade concrete and Fe-500 grade HYSD Bars. Assume the thickness of wearing coat as 80 mm. Sketch the details of reinforcements in the slab.

22.	Analyze the frame by cantilever method for a three bay two storeyed building having width of first bay is 4 m, second bay is 5 m, third bay is 3 m and height of first two stories is 3 m with lateral load of 100 kN and 80 kN, and bottom storey is 4.5 m with lateral load 50 kN.

(OR)

23.	Analyze the frame by portal method for a three bay two storeyed building having width of first bay is 5 m, second bay is 4 m , third bay is 3.5 m and height of first two stories is 3 m with lateral load of 120 kN and 100 kN, and bottom storey is 4.5 m with lateral load 75 kN.

24.	Write any four characteristics of yield lines and find the ultimate load of an isotropically reinforced square slab simply supported on all its edges and uniformly loaded. Use Virtual work method

(OR)

25.	Using equilibrium method design a rectangular slab 6 m x 4 m simply supported on all its edges. The live load is 5 kN/m2. The load factors are 1.5 and 2.2 for dead and live load respectively. The ultimate moment in the longer direction is half of the moment in shorter side. Use M25 grade concrete and Fe 415 HYSD Bars.