Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DESIGN OF STEEL STRUCTURES						       Time: 3 hours

Subject Code: CE252				       				      		       	       Maximum Marks: 100

#### Answer ALL questions

Use of Relevant BIS codes and Steel Tables are permitted. Assume suitable data wherever necessary.

PART – A  (10 x 1 = 10 MARKS)

1.	List out the different types of bolts used in structural members.

2.	What are called as battened columns?

3.	What is Web Crippling?

4.	What is meant by a Built up beam?

5.	Define Splice.

6.	Draw the neat sketch of a stiffened seated connection and name the parts.

7.	What are the loads to be considered for the design of roof trusses?

8.	What are purlins?

9.	What is meant by cold formed steel section?

10.	What are the loads to be considered for the design of a steel chimney?

PART – B  (5 x 3 = 15 MARKS)

11.	How do you calculate net section area of a staggered bolted connection?

12.	How do you find plastic moment capacity of a section?

13.	What are the functions of end bearing stiffeners and horizontal stiffeners?

14.	Discuss the factors to be considered in estimating wind pressure.

15.	What are the parameters to be considered for the strength of the light gauged steel column?

PART – C  (5 x 15 = 75 MARKS)

16.	Design a single angle section for a tension member of a roof truss to carry a factored tensile force of 225kN. The member is subjected to possible reversal of stress due to action of wind. The length of the member is 3m. Use 20mm shop bolts of grade 4.6 for the connection.

(OR)

17.	Design a laced column with two channels back to back of length 10m to carry an axial factored load of 1400kN. The column ends may be assumed to have restrained in position but not in direction at both the ends (hinged ends).

18.	Design a steel beam section for supporting the roof of a big hall for the following data and apply the usual checks. The compression flange of the beam is laterally supported throughout.

Clear span 						= 6.5m

End bearings						=150mm

C/c spacing of beams				=3m

Imposed load on beam   			= 10kN/m2

Dead load (inclusive of self weight)	= 4kN/m2

Restriction on beam depth			= 375mm

(OR)

[P.T.O]

19.	Design a simply supported beam of 10m effective span carrying a total factored load of 60 kN/m. The depth of the beam should not exceed 500 mm. The compressive flange of the beam is laterally, supported by floor construction. Assume stiff end bearing as 75 mm.

20.	Design a welded plate girder 24 m in span. It has to carry a uniform load of 100 kN/m throughout the span exclusive of self weight. Design the web, flange and connection between flanges and web. Also check for stresses.

(OR)

21.	Design an un- stiffened welded seat connection for a beam ISMB 250 @ 365.9N/m transmitting an end reaction of 110 kN due to factored loads to the flange of column ISHB 200 @ 392.4N/m. The seat angle is welded to column flange in a workshop.

22.	The trusses for a factory building are spaced at 4.5 m centers and the purlins are spaced at 1.8 m c/c. The pitch of the truss is 1/4 and the span of the roof is 10 m. The vertical load from roof sheets and fixtures is equal to 180 N/m2, while wind load on roof surface normal to the roof is 1200 N/m2. Design an angle purlin and check the stresses.

(OR)

23.	Design a Fink Type roof truss for an industrial building located at Chennai with following data,

Overall length of the building		= 48m

Overall width of the building		= 16m

Width (c/c of roof columns)		= 16m

C/C spacing of trusses				= 8m

Rise of truss						= 1/4 of span

Self weight of purlins				= 318N/m

Height of columns				= 11m

Roofing and side coverings		= 170N/m2

24.	Two channels 250 mm x 100 mm with bent lips are connected with webs to act as a column. The thickness of the plate is 3.0 mm and the depth of the lips is 25 mm. Determine the safe load carrying capacity if the effective length of the column is 6 m.

(OR)

25.	A column 10 m long has to carry a factored load of 1200 kN. The column is restrained in position but not in direction at both ends. Design a column with light gauge steel sections.