Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MECHANICS OF FLUIDS								      	       Time: 3 hours

Subject Code: CE277			       					               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What is an Ideal Plastic fluid?

2. 	Write the classifications of manometer.

3. 	The flow is said to be turbulent, if the Reynolds’s number is ________________.

4. 	Write the Continuity Equation for 3-D flow.

5. 	What is a Cipolletti Weir?

6. 	The discharge over a broad-crested weir is given by the equation _________________.

7. 	Write the Darcy’s formula for Energy loss due to friction.

8. 	What is the equation for Power transmitted through pipes?

9. 	Give the dimensions of Kinematic Viscosity.

10. 	What do you mean by Kinematic Similarity?

PART – B (5 x 3 = 15 MARKS)

11.	Explain 		a.	Newtonian Fluid 		b.	Non-Newtonian Fluid. Give examples for each.

12. 	Water is flowing through a pipe of 5 cm diameter under a pressure of 29.43 N/cm2 (gauge) and with mean velocity of 2 m/s. Find the total head or total energy per unit weight of the water at a cross-section, which is 5m above the datum line.

13. 	A pitot-static tube placed in the centre of a 300mm pipe line has one orifice pointing upstream and other perpendicular to it. The mean velocity in the pipe is 0.80 of the central velocity. Find the discharge through the pipe, if the pressure difference between the two orifices is 60mm of water. Take the coefficient of pitot tube as Cv=0.98.

14. 	Give the general characteristics of a laminar flow.

15. 	State Buckingham’s π Theorem. Why is this theorem considered superior over the Rayleigh’s method for dimensional analysis?

PART – C (5 x 15 = 75 MARKS)

16.	The space between two square flat parallel plates is filled with oil. Each side of the plate is 60cm. The thickness of the oil film is 12.5mm. The upper plate, which moves at 2.5m/sec, requires a force of 98.1 N to maintain the speed. Determine

a.	the dynamic viscosity of the oil in poise

b.	the kinematic viscosity of the oil in stokes, if the specific gravity of the oil is 0.95.

(OR)

17. 	A single column Manometer is connected to a pipe containing a liquid of specific gravity 0.9 as shown in the figure below. Find the pressure in the pipe if the area of the reservoir is 100 times the area of the tube for the manometer reading shown in figure. The specific gravity of mercury is 13.6.

[P.T.O]

40cm

18. 	In a free cylindrical vortex flow, at a point in the fluid at a radius of 200mm and at a height of 100mm, the velocity and pressures are 10 m/sec and 117.72 kN/m2 absolute. Find the pressure at a radius of 400mm and at a height of 200mm. The fluid is air having density equal to 1.24 kg/m3.

(OR)

19. 	Water is flowing through a pipe having diameters 20 cm and 10 cm at sections 1 and 2 respectively. The rate of flow through pipe is 35 lit/sec. The section 1 is 6 m above datum and section 2 is 4 m above datum. If the pressure at section 1 is 39.24 N/cm2, find the intensity of pressure at section 2.

20. 	An Orifice meter with orifice diameter 10 cm is inserted in a pipe of 20cm diameter. The pressure gauges fitted upstream and downstream of the orifice meter give readings of 19.62 N/cm2 and 9.81 N/cm2 respectively. Co-efficient of discharge for the meter is given as 0.6. Find the discharge of water through pipe.

(OR)

21. 	An external cylindrical mouthpiece of diameter 150mm is discharging water under a constant head of 6m. Determine the discharge and absolute pressure head of water at vena-contracta. Take Cd = 0.855 and Cc for vena-contracta= 0.62. Atmospheric pressure head = 10.3m of water.

22. 	Derive Darcy-Weisbach equation for loss of head due to friction in pipes.

(OR)

23. 	The rate of flow of water through a horizontal pipe is 0.3m3/sec. The diameter of the pipe is suddenly enlarged from 25cm to 50cm. The pressure intensity in the smaller pipe is 14 N/mm2. Determine 	a.	Loss of head due to sudden enlargement 	b.	Pressure intensity in the large pipe 	c.	Power lost due to enlargement.

24. 	A pipe of diameter 1.5m is required to transport an oil of specific gravity 0.90 and viscosity 3 x 10-2 poise at the rate of 3000 litres/sec. Tests were conducted on a 15cm diameter pipe using water at 20oC. Find the velocity and rate of flow in the model. Viscosity of water at 20oC = 0.01 poise.

(OR)

25. 	Define and explain the following:

a.	Froude’s Number

b.	Mach Number

c.	Distorted Model

d.	Un-distorted Model

e.	Hydraulic Similitude. 														(5 x 3)