Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: SURVEYING											      	      Time: 3 hours

Subject Code: CE280				       						              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What are the primary divisions of surveying?

2.	What is meant by contour line?

3.	The process of setting the theodolite exactly over the station mark is known as ______.

4.	What is meant by face right observation?

5.	Classify the methods of tacheometeric surveying.

6.	What is meant by orientation of plane table?

7.	What is meant by sight distance?

8.	If degree of a curve is 250, calculate the radius of the curve.

9.	What are the principles of surveying?

10.	Classify field book.

PART – B (5 x 3 = 15 MARKS)

11.	Define Sensitivity of a bubble tube.

12.	Differentiate between a Transit Theodolite and Non- Transit Theodolite.

13.	Mention the merits and demerits of movable hair method.

14.	Classify various types of curves.

15.	What is meant by well conditioned triangle?

PART – C (5 x 15 = 75 MARKS)

16.	The following consecutive readings were taken with a level and 5 meter leveling staff on continuously sloping ground at a common interval of 20 m: 0.385, 1.030, 1.925, 2.825, 3.730, 4.685, 0.625, 2.005, 3.110, 4.485. The RL of the first point was 208.125 m. Rule out a page of a level field book and enter the readings. Calculate the RL of the points by Rise and Fall method and also find the gradient of the line joining the first and last points.

(OR)

17.	What is a grade contour? Describe how to locate it in the field?

18.	List the Permanent adjustments of a theodolite. Explain any two.

(OR)

19.	Calculate the length of line EA and bearing for a traverse given below.

| Line | Length (m) | Bearing |
| --- | --- | --- |
| AB | 1015 | 118o 45’ |
| BC | 1022 | 42o 50’ |
| CD | 970 | 320o 42’ |
| DE | 850 | 254o 30’ |
| EA | ? | ? |

20.	State the three point problem and explain Bessel’s method of solving the same

[P.T.O]

(OR)

21.	Two sets of tacheometric readings are taken from an instrument station A, the reduce level of which was 100.06 m to a staff station B.

Instrument P – multiplying constant 100, additive constant 0.06m, staff held vertical.

Instrument Q – multiplying constant 90, additive constant 0.06m, staff held normal to the line of sight.

| Instrument | At | To | Ht of Instrument | Vertical Angle | Stadia Readings (m) |
| --- | --- | --- | --- | --- | --- |
| P | A | B | 1.5 m | 26o | 0.755, 1.005, 1.255 |
| Q | A | B | 1.45 m | 26o | ? |

What should be the stadia reading with the instrument Q?

22.	How is a simple curve set out by using one theodolite and one chain?

(OR)

23.	A simple circular curve is to have a radius of 573 m. The tangent intersects at chainage 1060 m and the angle of intersection is 12o. Find:

a.	Tangent distance.

b.	Chainage at beginning and end.

c.	Length of long chord.

d.	Degree of curve.

e.	Number of full and sub chords.

24.	What are the different methods of plotting a Traverse Survey? Explain in detail.

(OR)

25.	Explain the method of Reciprocal ranging.