Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: BIOCHEMICAL THERMODYNAMICS					       Time: 3 hours

Subject Code: CL221			       					               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is Entropy? How is it useful in Protein Folding?

2.	What are the limitations of First law of thermodynamics?

3.	Van der Waals equation results when the ideal gas equation is corrected to include the effect of

molecular interaction and the volume of the molecules. What are these correction terms?

4.	What is the application of Maxwell’s relationship?

5.	What is the criterion of phase equilibrium?

6.	What is meant by single reaction?

7.	What is the environmental application of biosensors?

8.	What is a Biosensor?

9.	Calculate the degree of reduction for methane.

10.	What is meant by catabolism and anabolism?

PART – B  (5 x 3 = 15 MARKS)

11.	Define the intensive and extensive properties with a simple example.

12.	Discuss the ideal gas law with appropriate equation.

13.	What is the effect of temperature on VLE?

14.	List out the different types of biosensors available for practical applications.

15.	Define the respiratory quotient with the help of simple stoichiometry.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the energy balance equation for the steady state steady flow process

(OR)

17.	Calculate U and  H for 1 kmol of water when it is vaporized at the constant temp of 373 K and the constant pressure of 101.33 KPa .The specific volume of liquid and vapor at these conditions are 0.0014 & 1.673 m3/kmol. For this change, heat in the amount of 1030 KJ is added to the water.

18.	Derive the four fundamental property relationship expressions using the energy property.

(OR)

19.	One kmol of CO2 occupies a volume of 0.381 m3 at 313 K .Compare the pressures given by:

a.	Ideal gas equation

b.	Van der waals equation, b= 4.28 x 10 –5 m3/mol; a = 0.365 Nm4/mol2

20.	Derive the relationship between free energy change and equilibrium constant.

(OR)

21.	In the synthesis of NH3, stoichiometric amount of N2 and H2 are sent to a reactor where the following reaction takes place. N2+3H2NH3

The equilibrium constant for the reaction at 675 K may be taken equal to 2x10-4.

Determine the percentage conversion of N2 to NH3 at 675 K and 20 bar.

[P.T.O]

22.	Discuss the applications of biosensors in industry and medical field.

(OR)

23.	Write a note on the following:

a.	Conducto-metric biosensors.													(7)

b.	Optic biosensors.																(8)

24.	Write a note on elemental balances and degrees of reduction with aid of simple stoichiometric expression.

(OR)

25.	Assume the experimental measurements for a certain organism have shown that cells can convert two-thirds (wt/wt) of the substrate carbon (glucose) to biomass. Calculate the stoichiometric co-efficients for the following biological reaction:

Glucose: C6H12O6 + a CO2 + b NH3 → c (C4.4 H7.3 N0.86 O1.2) + d H2O+ e CO2

Calculate the yield co-efficients YX/S and YX/O2.