Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ELECTRON DEVICES								      	       Time: 3 hours

Subject Code: EC201				     				              	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What are the types of Extrinsic Semiconductors?

2. 	Define Hall Effect.

3. 	What is the relationship between α and β?

4.	In the active region, while the collector-base junction is ________-biased, the base-emitter is       ________-biased.

5.	For the common-emitter amplifier ac equivalent circuit, all capacitors are ________ circuited.

6. 	The h parameter ________ is referred to as the reverse transfer voltage ratio.

7. 	For a JFET, the value of VDS at which Id becomes essentially constant is the ________ .

8. 	What are the regions in the VI characteristics of UJT?

9. 	What is a DIAC?

10. 	What is a photo diode?

PART – B (5 x 3 = 15 MARKS)

11. 	Define the term Transition capacitance.

12. 	What do you meant by Thermal runway?

13. 	Draw the T equivalent circuit for a CE transistor. Label the parts.

14. 	Differentiate BJT and UJT.

15. 	What is the Tunneling phenomenon?

PART – C (5 x 15 = 75 MARKS)

16. 	a.	With necessary diagrams, explain the energy band description of conductors, 	semiconductors and insulators.													(10)

b.	Write short notes on drift and diffusion currents.									(5)

(OR)

17. 	What is a PN junction diode? Explain the structure, operation and characteristics of PN junction diode.

18. 	What are the various transistor parameters and current components of a transistor? Explain in detail.

(OR)

19. 	Describe the static characteristics of a NPN transistor in CE configuration.

20. 	Draw the small signal model for CE amplifier and derive voltage gain, current gain, input impedance and output impedance.

(OR)

21. 	What are hybrid parameters? Draw the hybrid model for a two port network? How will you   obtain h-parameters from the characteristics of CE configuration? What are the limitations of h parameters?

[P.T.O]

22. 	With neat diagrams, explain the structure, operation and characteristics of JFET.

(OR)

23. 	Explain the construction, operation, volt ampere characteristics, and application of SCR, also

explain its two transistor model.

24. 	Write Short notes on:

a. 	Zener Diode.

b. 	Tunnel Diode.

(OR)

25. 	Explain the construction, operation, volt ampere characteristics, and applications of TRIAC.