Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: SOLID STATE CIRCUITS-I								       Time: 3 hours

Subject Code: EC205						       		      		       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is the efficiency of a half wave rectifier?

2. 	Mention any one advantage of a capacitor input filter.

3. 	Name the techniques used to maintain the operating point stable.

4. 	Why a fixed bias circuit is not commonly used?

5. 	What is transformer coupling?

6. 	Crossover distortion occurs in __________amplifier.

7. 	Noise reduction factor achieved by use of negative feedback is __________.

8. 	Negative feedback increases stability and bandwidth. True or False.

9. 	Give two examples for tank circuit oscillators.

10. 	A tuned amplifier has its maximum gain at a frequency of 2MHz and has a bandwidth of      	50KHz. Calculate the Q factor.

PART – B  (5 x 3 = 15 MARKS)

11. 	Draw the circuit of a series voltage regulator.

12. 	Define stability factor and give its expression for CE circuit.

13. 	Draw the complementary symmetry class B push pull amplifier circuit.

14. 	Compare positive and negative feedback.

15.  	A crystal has the following parameters: L=0.33H, C1=0.065pF, C2=1.0pF and        	R=5.5KΩ. Find the series resonant frequency and Q factor of the crystal.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the working principle of a full wave rectifier with a neat diagram and derive its DC load current, ripple factor, efficiency and transform utilization factor.

(OR)

17.	a.	Derive the expression of ripple factor for a full wave rectifier with L section filter.	(10)

b.	Write short notes on short circuit protection.                                                           	(5)

18.	a.	Explain the principle of fixed bias and collector to base bias circuits with neat diagrams.																						(10)

b.	Design a collector to base bias circuit for specified conditions. VCC=15 V, VCE=5V,     IC=5mA and β = 100.									 						(5)

(OR)

19.	a.	Design a self bias circuit for the specified conditions. VCC=12V, VCE=6V, IC=1mA,          			S=20, β = 100 & VE=1V.       				         			 					(10)

b.	Explain self bias circuit for JFET. 						             					(5)

20.	Draw a two stage RC coupled amplifier and explain its operation and derive its voltage gain.

(OR)

21.	a.	Prove that the maximum efficiency of class A transformer coupled power amplifier is 50%.

(10)

[P.T.O]

b.	A single ended class A amplifier has a transformer coupled load of 8Ω. If the transformer turns ratio (N1/N2) is 10, find the maximum power output delivered to the load. Take the zero signal collector current of 500mA.				            						(5)

22.	a.	Discuss the feedback topologies with necessary diagrams and write the expressions for 	input resistance and output resistance.											(12)

b.	Calculate the voltage gain, input and output resistances of a voltage series feedback 	amplifier having Av=300,Ri=1.5KΩ,Ro=50KΩ and β=1/15.                                    	(3)

(OR)

23. What is a differential amplifier? Explain its DC and AC operation and also derive the 	expression for CMRR.

24.  State the Barkhausen criterion for oscillation. Explain the working of a Wein bridge oscillator 	with a neat circuit diagram.

(OR)

25. 	With a neat sketch explain the operation of a single tuned amplifier and explain its frequency response.