Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DIGITAL SIGNAL PROCESSING						      	       Time: 3 hours

Subject Code: EC228			       					               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	List any two applications of DSP.

2.	What is a multidimensional signal?

3.	Define LTI system.

4.	What is wrapping effect?

5.	The Fourier transform of a discrete and aperiodic sequence is_______.

6.	State Parseval’s relation for discrete aperiodic signals.

7.	What is aliasing?

8.	Define quantization error.

9.	What is an anti-aliasing filter?

10.	Draw the 1st order IIR  digital filter.

PART – B  (5 x 3 = 15 MARKS)

11.	What are deterministic and random signals?

12.	Find the Z-transform of x[n]={1,2,3,4,5} and specify its ROC.

13.	How many complex multiplications and additions are required for computation of N-point DFT 	using FFT algorithm.

14.	State the Sampling theorem.

15.	Brief on the effects of windowing in FIR filter design.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Test whether the  signals are power, energy or neither energy nor power signals.

i.	x(n)= sin(π/4n)		ii.	x(n)=e2nu(n)        									(2x6)

b.	Give the relation between analog frequency and digital frequency.					(3)

(OR)

17.	a.	Determine the response of the system characterized by the impulse response,

h(n) = (1/2)nu(n)  to the input signal x(n)= 2nu(n).								(10)

b.	What are the advantages of digital signal processing over analog signal processing.	(5)

18. 	a.	Find the Z transform and ROC of the signal:										(5)

x(n)= [3(3)n – 4(2)n]u(n).

b.	Given the input-output relationship of a discrete time system,

y[n] - (1/4)y[n-1] = x[n]+2x[n-1],  find the system function H(z) and the impulse response 		h[n].																		(10)

(OR)

19. 	a.	Determine the system function H(z), impulse response h(n) of the LTI system defined by			y(n)=  x(n) +3x(n-1)+2y(n-1)-y(n-2).											(10)

b.	Find the Z-transform and ROC of the signal x[n]=cos0n u[n].						(5)

[P.T.O]

20.	Determine the 8 point DFT of the sequence x(n) ={1,1,1,1,1,1,0,0}	using DIT algorithm.

(OR)

21.	Compute the eight point DFT of the sequence below using DIF algorithm.

x(n)  =	{ 1 		0≤ n ≤7

{ 0      	otherwise

22.	a.	Compute the Nyquist sampling frequency for the signals:

i.	x(t) = 3 cos 4t 	ii.	x(t)= 4 sin c(3t/π)		iii.	sinc(4π/t)+sinc2(3π/t)			(10)

b.	Explain the sample and hold circuit with a neat diagram. 							(5)

(OR)

23. 	Design a Chebyshev filter with a maximum passband attenuation of 2.5dB at Ωp = 20 rad/sec and the stop band attenuation of 30dB at Ωs = 50 rad/sec.

24. 	a.	The system function is given by H(s)=2(s+1)(s+2)with T = 1 sec. Find H(z) by 	applying bilinear transformation.												(10)

b.	Obtain the direct form I realization for the system described by difference equation 	y(n)=0.5y(n-1)-0.25y(n-2)+x(n)+0.4x(n-1).										(5)

(OR)

25. 	a.	Explain the design of linear phase FIR filter using Hanning window.				(8)

b.	Explain the architecture of digital signal processor.								(7)