Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MICROPROCESSORS AND APPLICATIONS			      	      Time: 3 hours

Subject Code: EC238								       		    	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the need for ALE signal in 8085 Microprocessor?

2.	The 8085 has ___________ software interrupts.

3.	What is the function of Trap Flag (TP) in 8086?

4.	8086 has ___________ segments.

5.	8279 provides ___________ input modes for keyboard interface.

6.	Write the format of ICWI in 8259.

7.	What are the various display modes of 8279 keyboard / display controller?

8.	8255 is a ___________.

9.	How are high power devices interfaced with a microprocessor?

10.	Define step angle.

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate between HALT and HOLD states of 8085.

12.	Write the operations carried out when the instruction INAL, OOH and INAL, DX are executed by 8086.

13.	What are the ways by which the baud rate generation takes place?

14.	What is the function of EOI command in 8259?

15.	Name the interfacing devices / circuits commonly used in microprocessor based design and write their unique requirements.

PART – C  (5 x 15 = 75 MARKS)

16.	Using 8085 instruction set, write a program to generate a waveform having on time = 3 ms and off time = 2 ms.

(OR)

17.	With a neat diagram explain the interrupt architecture of 8085 microprocessor.

18.	Write an assembly language program in 8086 to search the largest data in a given array.

(OR)

19.	Explain the memory segmentation architecture with the supporting instructions.

20.	How does DMA controller help in fast data transfer?  Explain one such architecture with the required signals.

(OR)

21.	Explain the internal block diagram of USART (8251).

22.	Which interface could be selected for maintenance and enhancement of interrupts?  Explain in detail.

(OR)

23.	Explain the operation of 8295 printer controller.

24.	Discuss the interfacing of Temperature controller with 8085 CPU. Give the hardware interface diagram and flow chart.

(OR)

25.	Explain in detail the operation of a Traffic light controller.