Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: COMMUNICATION THEORY AND SYSTEMS			       Time: 3 hours

Subject Code: EC242			       						              	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Define Modulation.

2. 	Mention the types of Modulation Techniques.

3. 	In FM, Frequency deviation is proportional to phase of the modulation signal. True or False?

4. 	In TV Transmission, picture signals are transmitted by 	a. 	AM 	b.	 FM 	c.	PCM.

5. 	Mention the types of FM Transmitters.

6. 	Mention the functions of a Balanced Modulator.

7. 	AGC is used in a Radio Receiver to: 	a.	Improve Selectivity 	b.	Improve (S/N) R of the 	receiver 		c.	 Maintain the carrier level at the detector I/P constant: Choose the correct 	answer.

8. 	What are the types of Receivers?

9. 	Noise Figure of a multistage amplifier 	a.	Increases 	b.	Reduces 	c.	Does not 	change with decrease in number of stages. : Choose the correct answer.

10. 	List out the types of External Noise.

PART – B (5 x 3 = 15 MARKS)

11.	What is the need for Modulation?

12. 	Draw the Power Spectrum and Phasor representation of AM wave.

13. 	Differentiate between Envelope Detector and Synchronous Detector.

14. 	What are the basic functions of a Radio Receiver?

15. 	A radio reciver, connected to an antenna whose reactance is 50Ω has an equivalent resistance of 30Ω. Calculate the receiver Noise Figure in dB and its equivalent Noise Temperature.

PART – C (5 x 15 = 75 MARKS)

16.	Explain the block diagram of a Communication System.

(OR)

17. 	What is Modulation? Mention the need for modulation and list out the types of Modulation.

18.	A modulating signal 10 Sin (2π *103 t) is used to modulate a Carrier signal 20 Sin (2π *104 t).

Calculate: 	a.	Modulation index b. frequency of the side band components and their amplitudes	c.	Bandwidth		d.	Total power delivered to the load of 600Ω.

(OR)

19. 	Discuss in detail about Performance comparison of FM and AM systems.

20. 	Explain the working of a Balanced Modulator using Diodes and derive the O/P voltage.

(OR)

21. 	Explain the working of Foster Seeley Discriminator of FM Demodulator with necessary diagrams.

[P.T.O]

22. 	With a neat diagram explain the function of an AM Super heterodyne receiver.

(OR)

23. 	Discuss in detail about Pre-emphasis and De-emphasis in FM.

24. 	Short notes on:

a.	Thermal Noise 																(10)

b.	Shot Noise																	(5)

(OR)

25. 	Discuss about Noise in AM systems and obtain the expression for Output to Input SNR.