Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DIGITAL SIGNAL PROCESSING							       Time: 3 hours

Subject Code: EC245			       					             		       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	How does the spectrum of DT signals differ from CT signals?

2.   	How many additions and multiplications are required for radix-2 FFT?

3.   	Why is FIR filter stable?

4.   	Illustrate Gibb’s phenomenon.

5.   	Write the equation for IIR filters.

6.   	Draw the frequency response of Butterworth low pass filter.

7.   	What is the necessity for floating point notation?

8.   	What is overflow in adders?

9.   	What makes a filter adaptive?

10.  	State the advantages of pipelining.

PART – B (5 x 3 = 15 MARKS)

11.  	Find the circular convolution of : x(n) = {1, 1, 1, 1},  h(n) = {4}.

12.  	Draw the linear phase structure for an FIR filter of length 5.

13.  	What is the drawback of impulse invariance transformation?

14.  	Derive the quantization noise power.

15.  	What is the memory available on TMS320C5416?

PART – C (5 x 15 = 75 MARKS)

16.  	Find the 8-point DFT of the given signal using DIT FFT algorithm:

x[n] = {1, 2, 3, 4, 5, 6, 7}

(OR)

17.  	Using DFT and IDFT, determine the response of the FIR filter with impulse response:

h(n) = {1, 2, 3} to the input sequence x(n) = {1, 2, 2, 1}.

18.  The desired frequency response of a LPF is given by:

Hd(ejω)= e−j2ω, −π4 ≤ ω ≤ π4 = 0 , otherwise

Determine the filter coefficients.  If these coefficients are passed through a rectangular        	window of length 5, find the new frequency response of the filter.

(OR)

19.  	Consider an FIR filter with system function,  H(z) = 1 + 2.88z−1 + 3.4048z−2 + 1.74z−3 + 0.4z−4.

Sketch the direct form and lattice realization of the filter.

20.  a.	Explain the bilinear transformation method of designing digital filters.  Why is prewarping 	required in such filters?  														(10)

b.	Using the above method, convert the following analog filter into digital filter:		(5)

H(s) = s+0.1(s+0.1)2 + 16												[P.T.O]

(OR)

21.  	Obtain the direct form I, direct form II, cascade and parallel structures for the following        	system:

H(z) = 2(1−z−1)(1+2z−1+z−2)(1+0.5z−1)(1−0.9z−1+0.81z−2)

22.  	Explain limit cycle oscillations in recursive systems. Illustrate with an example.

(OR)

23.  	Explain the effect of coefficient quantization error in FIR and IIR digital filters.

24.  a.	Derive the Wiener-Hopf equations for optimum FIR filter.							(9)

b.	Write an ALP that initializes a memory location with any given value using:			(6)

i.	Absolute addressing

ii.	Direct addressing

iii.	Indirect addressing

(OR)

25.  	With neat diagrams, explain the architecture of TMS320C5416 processor.