Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DIGITAL COMMUNICATION							       Time: 3 hours

Subject Code: EC249			       						             	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Relate sampling rate with Nyquist rate.

2.	How is the quantization error compared with the mean square quantization error?

3.	Which of the modulation schemes eliminate the ambiguities of BPSK?

4.	________ encoding affects a reduction of maximum frequency in comparison to the maximum frequency of unencoded data.

5.	Write the formula for finding the probability of error of non – coherent FSK.

6.	Define matched filter.

7.	Find out the entropy for an L message sequence of M different messages with their probabilities of occurrences.

8.	State Shannon – Hartley theorem.

9.	What are the two specific needs of spread spectrum?

10.	Define Processing gain.

PART – B  (5 x 3 = 15 MARKS)

11.	What is Inter symbol Interference? Refer a solution to overcome the same.

12.	Show that the errors in differential encoded PSK occur in pairs.

13.	Draw the block diagram of a coherent system of signal reception.

14.	An analog signal is band limited to B Hz, sampled at the Nyquist rate and the samples are quantized into 4 levels. The quantization levels are assumed independent and occur with probabilities p1=p4= 1/8, p2=p3=3/8. Find the information rate of the source.

15.	List out the properties of the PN sequence.

PART – C  (5 x 15 = 75 MARKS)

16.	Show the block diagram of PCM communication system. Discuss the encoder and decoder designs in detail.

(OR)

17.	Why do we need Delta Modulation? Compare its working and performance with ADM.

18.	Derive the signal space representation of QPSK with the constellation.

(OR)

19.	How does MSK differ from QPSK? With neat waveforms analyze the performance of MSK.

20.	Mention the main the purpose of optimum filter in the receiver. Calculate its transfer function.

(OR)

21.	What is a four phase QPSK? With a phasor diagram and a correlation receiver, obtain its probability of error.

[P.T.O]

22.	a.	Describe briefly on Bandwidth – SNR trade off.   								(6)

b.	Write the steps involved in generating block codes.								(9)

(OR)

23.	With a sample coder, generate the convolution code and perform the decoding using Tree structure.

24.	Analyze the effect of single – tone interference in DSSS system. Derive the effective jamming power.

(OR)

25.	Show the spectrum of FHSS / BFSK and the diagram representing its modulation / 	demodulation system. Explain the detection process.