Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DIGITAL SYSTEM DESIGN USING VHDL				       Time: 3 hours

Subject Code: EC250			       						          	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What is meant by ASM chart?

2. 	What is the main advantage of a bit slicing processor?

3. 	Write some RTL notations commonly used.

4. 	What are the three type of modeling in VHDL?

5. 	What are the different types of semicustom design?

6. 	In VHDL which modeling uses a set of concurrent signal assignment statements?

7. 	Name any three FPGAs.

8. 	What is the programming element used in Xilinx FPGA?

9. 	What are the various types of port declarations in VHDL?

10. 	The output of “1101” & “1001”is_______.

PART – B (5 x 3 = 15 MARKS)

11. 	Compare PAL with PROM.

12. 	What is meant by ROM simulation?

13. 	What is meant by a test bench?

14. 	What is meant by sensitivity list in VHDL?

15. 	Write the VHDL code for a half subtractor.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Describe the construction of a PLA with a neat diagram.							(7)

b. 	Implement a full adder and full subtractor using a PAL. 							(8)

(OR)

17.	a. 	Describe the construction of PAL with a neat diagram.							(7)

b. 	Realize the following functions using a PAL:

f1(x,y,z)=∑(1,2 4 5 7) ; f2(x,y,z)= )=∑(0,1,3,5,7).								(8)

18. 	a. 	Compare hardwired control and micro programmed control.						(8)

b. 	Draw an ASM chart for a mod 8 binary up-down counter.							(7)

(OR)

19. 	a. 	Write a short note on advanced Boolean expression language.						(6)

b. 	Describe the organization of a micro programmed control unit with a neat diagram.	(9)

20. 	a. 	Explain about the FPGA 4000 series CLB.										(8)

b. 	Discuss about semicustom design and full custom design.							(7)

(OR)

21. 	a. 	Explain about FPGA 4000 series I/O block.										(8)

b. 	Describe about Actel series FPGAs.											(7)

22   a. 	Discuss the VLSI design flow process.											(6)

b. 	Explain data operators in VHDL with examples.									(9)

[P.T.O]

(OR)

23. 	a. 	What are the various data types in VHDL?										(9)

b. 	Explain about component instantiation.											(6)

24.	a. 	Write a VHDL program for a JK flip flop using behavioral modeling.				(8)

b. 	Compare inertial delay and transport delay in VHDL.								(7)

(OR)

25   	a. 	Write a VHDL code for a 4:1 MUX using structural modeling.						(9)

b. 	Write a test bench for the above program.										(6)