Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: SATELLITE COMMUNICATION						      	       Time: 3 hours

Subject Code: EC251			       					               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define apogee.

2.	What is meant by payload?

3.	Define S/N ratio.

4.	What is a polar antenna?

5.	What is meant by a transponder?

6.	What is meant by frequency reuse?

7.	What are the two types of multiple access techniques?

8.	Give the two types of CDMA.

9.	Give two applications of INTELSAT.

10.	What is VSAT?

PART – B  (5 x 3 = 15 MARKS)

11.	State Kepler’s first law.

12.	A satellite downlink at 12 GHz operates with a transmit power of 6 W and an antenna gain of

48.2 dB. Calculate the EIRP in dBW.

13.	Write the equations of C/N ratio.

14.	Define satellite switched TDMA.

15.	What are the three main orbits considered for mobile communication?

PART – C  (5 x 15 = 75 MARKS)

16.	Describe the terms of Earth orbiting satellites.

(OR)

17.	Explain briefly the Telemetry tracking and Command subsystem.

18.	An earth station transmits at 5.62 GHz from an antenna of 6 m.  The transmitter generates an output of 8 KW.  The satellite is 39920 KM from the earth station. The efficiency of transmitting antenna being 0.7 calculate:

a.	Path loss  						b.	Transmitting antenna gain.

c.	EIRP 							d.	Received power at the satellite.

(OR)

19.	Derive the carrier to noise ratio and G/T of a receiving system.

20.	Write about frequency reuse concept and orthogonal polarization.

(OR)

21.	Write short notes on:

a.	Terrestrial interference.          		b.	Cross polarization interference.

22.	a.	Explain the principle of TDMA concept.										(10)

b.	Write short notes on Traffic Burst.												(5)

(OR)

[P.T.O]

23.	Explain the principle of CDMA satellite system. How does it compare with TDMA and FDMA?

24.	With a neat diagram explain satellites for mobile communication.

(OR)

25.	With a necessary diagram explain about the INMARSAT system.