Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DIGITAL SYSTEM DESIGN USING VHDL				       Time: 3 hours

Subject Code: EC258							       		    		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the difference between PAL and PLA?

2.	Give some examples for PLD.

3.	List some FPGA vendors.

4.	What do you mean by CPLD?

5.	Why go for HDL?

6.	Name the different abstract levels of HDL.

7.	Define simulation.

8.	Compare simulation and synthesis.

9.	Explain Testbench.

10.	Differentiate between Portmap and Generate.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the basic structure of a PLA.

12.	What is the difference between semi custom and full custom IC design?

13.	When should one use ‘signal’ and when should one use ‘variable’?

14.	Compare concurrent signal assignment statements and sequential signal assignments.

15.	Write the HDL code for a Half adder (2bit) using structural modeling.

PART – C  (5 x 15 = 75 MARKS)

16.	Realize the following functions using a PLA

F1	=	Σm(2,3,5,7,9,0,10,11,13,15)

F2	=	Σm(2,3,5,6,7,10,11,14,15)

F3	=	Σm(6,7,8,9,13,14,15)

(OR)

17.	Realize the function I1I2' +I1'I2using PAL segment compare PAL with PLA.

18.	Describe Xilinx XC 3000 series logic cell and explain CLB, input and output blocks, programmable interconnects.

(OR)

19.	Describe Xilinx XC 4000 series logic cell and explain CLB, I/O Block and PIC.

20.	Discuss the various Data types, Data operators and Data objects in VHDL.

(OR)

21.	Design 4 bit adder using generate statement.

22.	a.	With the use of CASE statement design a 2:1 MUX using behavioral modeling.		(5)

b.	Develop a behavioural and dataflow model for a T flip flop.						(10)

(OR)

23.	Discuss about the selected signal assignment and write an HDL for a 4 bit ripple carry counter.

[P.T.O]

24.	Write an HDL code for a Traffic light controller.

(OR)

25.	a.	Compare Behavioural, Data flow and Structural modeling.

b.	Write an HDL code for an 8:1 MUX using two 4:1 MUX and one 2:1 MUX using 	structural modeling.