Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ELECTRONIC INSTRUMENTATION					       	       Time: 3 hours

Subject Code: EC261		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	The voltage of a circuit is measured by a voltmeter having input impedance comparable with the output impedance of the circuit, thereby, causing error in voltage measurement. This error may be called

a.	Gross error 		b.	Random error 	c.	error caused by misuse of instrument 			d.	Error by loading effect.

2.	The range of Kelvin’s double bridge is:

a.	0.1 µΩ to 1 Ω 		b.	0.01 µΩ to 0.1 Ω		 c.	0.1 µΩ to 1 MΩ

d.	greater than 1 MΩ

3.	Strain gauge is a

a.	Zero order instrument  	b.	First order instrument 		c.	No order instrument

d.	Second order instrument

4.	In an LVDT, the two secondary windings are connected in differential to obtain

a.	Higher output voltage.

b. 	An output voltage which is phase sensitive i.e. the output voltage has a phase which can lead us to a conclusion whether the displacement of the core took place from right to left or from left to right.

c. 	In order to establish the null or the reference point for the displacement of the core.

d. 	Both b and c.

5.	A vertical amplifier for a CRO is designed for

a.	Only a high gain   	b.	A constant gain 		c.	Only a broad bandwidth

d.	all the above.

6.	Discuss in brief the Lissajous patterns.

7.	Explain about Distortion.

8.	Give some application of X-Y recorder.

9.	Distinguish between analog and digital voltmeter.

10.	Mention the relation between Frequency and Time period.

PART – B (5 x 3 = 15 MARKS)

11.	Give the difference between repeatability and reproducibility in static characteristics.

12.	A capacitive transducer is made up of two concentric cylindrical electrodes. The outer diameter of the inner cylindrical electrode is 3 mm and the dielectric medium is air. The inner diameter of the outer electrode is 3.1mm. Calculate the dielectric stress when a voltage of 100 V is applied across the electrodes. The length of electrode is 20 mm. Calculate the change in capacitance if the inner electrode is moved through a distance of 2 mm.

13.	Explain the function of a Spectrum analyzer.

14.	Explain the application of a Digital storage oscilloscope.

15.	Give a note on the IEEE488 standard.

[P.T.O]

PART – C (5 x 15 = 75 MARKS)

16. 	Derive the general equations of balance for an A.C. bridge. Prove that two conditions i.e. for magnitude and phase have to be satisfied if a bridge is to be balanced. Also derive the equation for balance in the case of an Anderson bridge with the phasor diagram.

(OR)

17.	Explain the following terms.

a.	Hysteresis.		b.	Linearity.		c.	Static sensitivity.		d.	Accuracy.

e.	Precision.

18.	Discuss the properties of Piezo electric crystals. Also explain charge and voltage sensitivity with necessary derivations.

(OR)

19.	Describe the working principle and constructional details of a Thermistor. Discuss the various materials used. Also differentiate with RTD.

20.	Describe how the following measurements can be made with the use of CRO:

a.	 Frequency 		b.	 Phase.

(OR)

21.	Explain the working principle of X-Y recorders and Strip-chart recorders.

22.	Draw and explain the block diagram of the digital method of measuring frequency and period between two signals.

(OR)

23.	Draw and explain the Computer controlled test systems.

24.	Discuss the following generators:

a.	Audio frequency generator.			b.	Pulse generator.

(OR)

25.	Discuss the following analyzers:

a.	Wave analyzer.						b.	Distortion analyzer.