Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: COMPUTER ARCHITECTURE						      	       Time: 3 hours

Subject Code: EC263			       					              	              Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Why do we need a biased exponent representation in floating point numbers?

2.	Write Booth’s expression for a signed binary number 110101.

3.	Compute the number of clock pulses required to process 100 tasks in an eight segment pipeline.

4.	Define pipeline’s performance/cost ratio and speedup performance.

5.	What are the different phases of the basic computer instruction cycle?

6.	Define vector interrupt.

7.	What are the basic functionalities that should be performed by an I/O interface?

8.	What is meant by bus arbitration?

9.	Differentiate synchronous and asynchronous bus transfers.

10.	What is SCSI?

PART – B  (5 x 3 = 15 MARKS)

11.	What is the von Neumann architecture? How is it different from the Harvard architecture?

12.	What is a Bit-slice processor?

13.	What is superscalar processing?

14.	Draw a CMOS memory cell structure.

15.	List out the hand shake signal support of DMA controllers.

PART – C  (5 x 15 = 75 MARKS)

16.	Give a detailed note on the von Neumann architecture with a relevant diagram.

(OR)

17.	Explain the terms involved in the basic performance equation of a computer. Also discuss how you will measure the performance of a computer.

18.	Multiply the following pair of signed 2’s complement numbers using Booth’s  algorithm where 110101 is the multiplicand and 011011 is the multiplier. Repeat the problem using bit-pairing of multipliers.

(OR)

19.	Divide 1000 by 11 using 		a.	restoring algorithm 	b.	non-restoring algorithm.

20.	What are the major difficulties that deviate the instruction pipeline from its normal operation? Describe with examples.

(OR)

21.	How will you modify the 3-bus structured internal datapath to suit a pipelined design?

22.	Explain Direct Memory Access mode of data transfer with a neat sketch.

(OR)

23.	Analyze the different mapping functions of a cache memory.

24.	Briefly explain the interrupt initiated I/O and describe how the priority is assigned to the interfacing devices.

(OR)

25.	Explain in detail about the standard USB I/O interfaces.