Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: TRANSMISSION LINES AND WAVE GUIDES			       	       Time: 3 hours

Subject Code: EC280			       						                     Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is a Flat line?

2.	Name an Unbalanced version of the transmission line. Mention an application of it.

3.	Write any two potential applications of a Half wave Transformer.

4.	What are the features of a Double stub tuner?

5.	A hollow waveguide behaves like a _________.

6.	Write the relation between vp and vg .

7.	A 10 GHz wave is propagating in a waveguide having a wall separation of 4 cm. The largest

number of half waves of electric intensity possible in this waveguide  is _________.

8.	_________ mode does not exist in waveguides. Why?

9. 	_________ mode offers widest tuning range in cylindrical cavity resonators.

10.	List the salient features of circular waveguides.

PART – B (5 x 3 = 15 MARKS)

11.	What are the different types of transmission lines? Briefly comment on them.

12.	Derive the relation between VSWR and Reflection coefficient.

13.	When a wave of 6 GHz propagates in parallel conducting plates separated by 3cm, find the

vp and vg  of the wave for the dominant wave.

14.	Explain how TE10 mode can be excited only in one direction of a rectangular waveguide.

15.	Define   a.	Wave meter      b. 	Q of a cavity resonator.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Derive the Telegraphic equations of transmission lines.						 	(10)

b.	Explain the characteristics of distortion less transmission line and telephone cables.	(5)

(OR)

17.	a.	Define and explain:   i. Characteristic impedance  ii. Transfer impedance   iii. Propagation

constant  iv. Reflection coefficient.							   					(8)

b.	Derive the input impedance of a lossless line. For a shorted section of 75 ohm transmission

line, l= λ/4.  Find the input impedance assuming α = 0.					   		(7)

18.	a.	Explain the characteristics of a Smith chart.						   				(5)

b.	Derive the Smith chart equations.							  					(10)

(OR)

19.	a.	Derive the input impedance for a loss less quarter wave transformer terminated by an 			impedance ZL.										     						(8)

b.	Explain the principles of impedance matching using Single stub tuner and double stub tuner with diagrams. Differentiate SST from DST.						    		(7)

[P.T.O]

20.	a.	Explain the characteristics of TEM waves. Derive their propagation parameters. 	    	(9)

b.	When a wave of 6 GHz is to be propagated between two parallel conducting plates separated by 60 mm, find the modes that will propagate through the guide.			(6)

(OR)

21.	a.	Explain the characteristics of TE waves. Derive the field components of TE waves in 			parallel planes.																(10)

b.	Give an account on characteristics of Uniform plane waves.			    			(5)

22.	a.	Derive the field equations for TM waves in rectangular hollow waveguides.		  	(10)

b. 	Discuss the different methods of excitation of modes in rectangular waveguides.	    	(5)

(OR)

23.	a.	Define and explain:

i.	 	Evanescent Mode       ii.	Degenerate modes

iii.		Surface impedance     iv. 	Skin depth.						    			(8)

b.	Design a rectangular waveguide (determine dimensions a and b), so that the cutoff frequency for the TE10  mode is 14 GHz and the cutoff frequency  for the TM11  mode is 30 GHz. 																		(7)

24.	a.	Explain the significance of Bessel functions in circular waveguide analysis, with an 	example.										    	      						(5)

b.	Mathematically prove that the dominant mode for a circular waveguide is TE11.  	     	(10)

(OR)

25.	a.	Design a rectangular cavity to have a resonant frequency of 9.8 GHz, having dimensions

a=d and b= a/2.										      						(7)

b.	The resonant frequency of a cavity is 8.8 GHz. It is critically coupled to an external circuit.

The measured bandwidth with loading is 4 MHz. Calculate the loaded and unloaded Qs.																				(8)