Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ANTENNAS AND WAVE PROPAGATION				      	       Time: 3 hours

Subject Code: EC282			       						             	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Hertzian dipole is ________.

2.	Write down the expression for retarded vector potential.

3.	_______antenna produces circularly polarized waves.

4.	What is dBm?

5.	________ antenna is also called as super gain or super directive antenna.

6.	The condition for frequency independence is ________.

7.	______antenna is complementary to slot antenna.

8.	Examples of secondary antennas: ________, ________.

9.	What is multihop propagation?

10.	Define fading.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the radiation resistance of a current element whose overall length is λ /50?

12.	Differentiate broadside and end fire array.

13.	Give the expressions for design ratio, spacing factor and frequency ratio of log periodic      	antenna.

14.	State Huygen’s principle.

15.	Find the range of LOS (Line of Sight) system when the receive and transmit antenna heights 	are 10m and 100m respectively.

PART – C  (5 x 15 = 75 MARKS)

16. 	Derive the fields radiated from a half wave dipole antenna. Find the power radiated and 	radiation resistance of a half wave dipole.

(OR)

17.	Explain the use of capacitance hat and loading coil for short antennas.

18.	Derive the radiated fields of a small loop antenna. Find its radiation resistance.

(OR)

19.	a.	Explain the principle of pattern multiplication.                                 					(10)

b.	A transmitting antenna having effective height of 61.4 m takes a current of 50 A, at a              		wavelength of 625 m. Find Radiation resistance, power radiated by the antenna and 				antenna 	efficiency for antenna resistance RA=50.                    						(5)

20.	a.	Give the design details of a rhombic antenna.                                        				(10)

b.	Discuss the radiation pattern for a travelling wave antenna of different lengths.   		(5)

(OR)

21.	a.	Discuss about self impedance and mutual impedance.                           				(5)

b.	Explain the construction and operation of a Yagi uda antenna.               				(10)

[P.T.O]

22.	Derive the various field components radiated from circular aperture.

(OR)

23.	a.	Write about feed systems of reflector antennas.                                    				(8)

b.	Describe the principle of lens antennas.                                                   				(7)

24.	a.	Explain attenuation characteristics of ground wave propagation.         				(8)

b.	Discuss about diversity reception.                                                                    			(7)

(OR)

25.	Describe how the ionospheric layers D, E, F1 and F2 are formed and how they affect the        	propagation of radio waves.