lReg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DIGITAL ELECTRONICS								      	       Time: 3 hours

Subject Code: EC284						       			    		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Convert (FF)16 into decimal.

2.	Simplify the Boolean expression (+B)(A+B) .

3.	What is a priority encoder?

4.	Draw the logic diagram of a full-adder circuit

5.	Write down the equation for maximum count capability of a counter.

6.	What is a ring counter?

7.	Define a bit.

8.	What do you mean by lock out condition?

9.	What are the different types of digital logic families?

10.	What is the disadvantage of DRAM?

PART – B  (5 x 3 = 15 MARKS)

11.	Given the two binary numbers X = 1010100 and Y = 1000011, perform the subtraction

a.	X –Y 	b.	Y – X   using 2's complements.

12.	What do you mean by fundamental mode circuit?

13.	What are hazards?

14.	Compare SRAM and DRAM.

15.	What are the advantages of TTL circuits?

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	State and prove De-Morgan’s Theorem.											(7)

b. 	Reduce 	i.	AB + (AC)' + AB’C (AB + C) 		ii.	ABC + ABC' + AB'C + A'BC.	(8)

(OR)

17.	Reduce the following function using Tabulation method.

F = Σ m (0, 1, 2, 3, 4, 6, 8, 10, 11)

18.	a. 	Implement a Full-Adder circuit with two half-adders and an OR gate. 				(8)

b.	Write short notes on Decoders and design a 3 to 8 decoder. 						(7)

(OR)

19.	Design a 2 bit comparator using gates.

20. 	What do you mean by race around condition? How is it eliminated using a Master slave flip flop?

(OR)

21. 	Design and explain the working of mod-7 counter.

[P.T.O]

22. 	Design a clocked asequential circuit that operates according to the state diagram given using D Flip flop.

(OR)

23. 	Design a serial parity generator using asynchronous sequential circuit.

24. 	a.	Draw the schematic of an RTL NOR gate and explain its operations. 				(10)

b. 	What limits the speed of a TTL Gate? How can the speed of TTL gate be enhanced?	(5)

(OR)

25. 	a. 	With a neat illustration, explain the construction and working principle of a TTL   inverter.

(12)

b. 	How do open collector outputs differ from the Totem Pole outputs?    		 		(3)