Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: SOLID STATE CIRCUITS									       Time: 3 hours

Subject Code: EC288			       					             		       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	The process of conversion from AC to DC is called _________.

2.	What is ripple factor?

3.	For good stability the Q – point should be _______.

4.	The field effect transistor is _________ device.

5.	List out the applications of RC coupled amplifier.

6.	The cross over distortion takes place in ______ amplifier circuits.

7.	What is positive feedback?

8.	Define common mode rejection ratio.

9.	What is the condition for sustained oscillation?

10.	Write down the resonance frequency of the tuned circuit.

PART – B (5 x 3 = 15 MARKS)

11.	Draw the circuit diagram of L- section filter and explain its function briefly.

12.	Define the following:  a.	Thermal run away 	b.	Q- point 	c.	Stability factor.

13.	The tank circuit contains an inductance of 1mH. Find out the range of capacitor value if the resonance frequency ranges from 540 KHz to 1650 KHz.

14.	Explain the effect of negative feedback on voltage series feedback amplifier.

15.	What is an Oscillator? State Barkhausan criterion.

PART – C (5 x 15 = 75 MARKS)

16.	Calculate the Vdc, Idc, Vr(rms), Ir(rms) through a 1KΩ load connected to a half wave rectifier circuit. The Vrms value is 230V. Find out the ripple factor.

(OR)

17.	Draw the circuit diagram of current series voltage regulator and explain its operation.

18.	Draw the fixed bias circuit for transistor and explain its operation in terms of stability. Give the advantages and disadvantages for fixed bias circuit.

(OR)

19.	a.	Discuss about self biasing in FET with a circuit diagram. 							(10)

b.	Write short notes on Voltage feedback biasing. 									(5)

20.	Explain about the operation of transformer coupled amplifier with a circuit diagram. Give the 	advantages and limitations for the same.

(OR)

21.	Describe about Class B push pull amplifier operation, load line characteristics and power calculation with diagram.

22.	Describe the input and output resistance for the following negative feedback amplifier.

a.	Voltage shunt 		b.	Current shunt

(OR)

[P.T.O]

23.	Draw the circuit diagram of a Differential amplifier and its operation.

24.	Discuss about the working principle and operation of RC phase shift oscillator. Explain how the Barkhausan criterion is satisfied in this circuit?

(OR)

25.	Explain about the operation of double tuned amplifier with a diagram and list out the applications.