Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: PULSE AND WAVE SHAPING CIRCUITS					       Time: 3 hours

Subject Code: EC289			       						         	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What do you mean by peaking circuit?

2.	RL circuits are rarely used when a ___________ time constant is required.

3.	The diodes used in a bistable multivibrator to maintain a constant output swing are called as _____________ diodes.

4.	When should one use the unsymmetrical triggering?

5.	Why is a monostable multivibrator called a gating circuit?

6.	A ___________ multivibrator can be used as a voltage-to-frequency converter.

7.	Define displacement error in sweep circuits.

8.	Bootstrap time base generators produce a ___________ sweep.

9.	What do you mean by a blocking oscillator?

10.	A sampling gate which can handle the input signal excursions of both the polarities is termed as ______________.

PART – B (5 x 3 = 15 MARKS)

11.	Design a diode clamper to restore the bottom peaks of the input signal to zero level. Use a silicon diode with Rf = 50 Ω and Rr = 400 k Ω. The frequency of input voltage is 5 kHz. Assume the time constant as 20T.

12.	Discuss the steps to improve the resolution of commutating capacitors.

13.	Draw a gated astable multivibrator.

14.	Derive the transmission error of the exponential sweep circuit in a time base generator.

15.	List the advantages and disadvantages of a unidirectional diode gate.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Explain how the low-pass RC circuit acts as an integrator.							(3)

b.	Explain working of an emitter-coupled clipper with the help of a neat diagram.		(7)

c.	State and prove the clamping circuit theorem.									(5)

(OR)

17.	Derive an expression for the output of a high-pass RC circuit excited by:	a. sinusoidal input b.	a ramp input 		c.	an exponential input.	         							(5+5+5)

18.	What is a bistable multivibrator. Explain the fixed -bias bistable multivibrator with a neat circuit diagram and also discuss the loading effects.

(OR)

19.	Design a Schmitt Trigger circuit to have Vcc = 12 V, UTP = 6 V, LTP = 3 V, using two silicon n-p-n transistors with hFE(min) = 60. Assume IC2 = 2 mA and I2 = IC2/10.

20.	Discuss the operation of the collector-coupled monostable multivibrator with a circuit diagram and waveforms. Derive the expression for gate width.

(OR)

[P.T.O]

21.	With the help of a circuit diagram and waveforms explain the operation of an emitter –coupled astable multivibrator and derive the expression for the frequency of oscillation.

22.	a.	Explain the working of a transistor miller time-base generator with circuit diagram.	(8)

b.	Find the component values of a bootstrap sweep generator, given, VCC = 18 V,  IC(sat) = 2 mA, and  hfe(min) = 30.	  						  								(7)

(OR)

23.	a.	With the help of a neat circuit diagram, explain the working of a transistor current time-	base generator.								 	  							(8)

b.	Discuss the Sweep circuit operation using UJT with neat sketches.		  			(7)

24.	Explain the working of a diode-controlled astable blocking oscillator with the help of a neat circuit diagram and waveforms. Derive an expression for the period.

(OR)

25.	a.	Explain the operation principle for bidirectional diode sampling gate and give its 	advantages.																	(9)

b.	Describe the operation of a four-diode sampling gate circuit and compare its advantages over two-diode sampling circuit.												(6)