Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: BASIC ELECTRICAL ENGINEERING					       Time: 3 hours

Subject Code: EE101/DEE101				       			              	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	The total charge entering a terminal is given by q=5tsin4πt  mC. The current at time t = 0.5 seconds is ____________.

2.	Which type of armature winding is suitable for high-voltage and low-current DC machines?

3.	Write the equation of an alternating sinusoidal voltage, of 50 Hz frequency, with maximum voltage of 200 V.

4.	The flux linking both the primary and secondary windings in a transformer is __________.

5.	In short transmission lines, the effect of capacitance is _______________.

6.	The lines feeding the power from the distribution transformer to the distributors are called ____________.

7.	What is the use of counter emf in a DC motor?

8.	The expression for finding the frequency of rotor current in a 3-phase induction motor is ____________.

9.	The two types of moving iron measuring instruments are _________________.

10.	T.R.S. wiring means _____________.

PART – B (5 x 3 = 15 MARKS)

11.	State the applications of DC shunt, series, and compound generators.

12.	For an alternating quantity, explain about phase and phase difference.

13.	Mention the classification of cables.

14.	Draw the characteristics of DC shunt and series motors.

15.	Explain briefly about damping torque in measuring instruments.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Define the following electrical quantities:		                                        			(5)

i. 	Electric potential				ii.	Electric power

iii. 	Temperature coefficient of resistance.

b.	Draw and explain the characteristics of DC shunt and series generators.	 		(10)

(OR)

17.	a.	Explain the working principle of a DC generator and describe its construction.    		(10)

b.	Derive the EMF equation of a DC generator.			                            				(5)

18.	a.	A current has the following steady values in amperes for equal intervals of time changing 		suddenly from one value to the next:

0, 5, 10, 20, 50, 60, 50, 20, 10, 5, 0, 5, 10, etc.

Calculate:	i. 	Average value	ii. 	RMS value	iii. 	Form factor	iv.	Peak factor.(7) 	b.	Describe the constructional details of a synchronous generator.	                			(8)

(OR)

19.	Describe the constructional details, types, and principle of operation of a two-winding transformer.

[P.T.O]

20.  Describe the schematic arrangement of a hydro-electric power generating station and explain 	the function of each component.

(OR)

21.	a.	What are the properties of conductors and insulators?			                			(7)

b.	Compare overhead and underground distribution systems.		                			(8)

22.	a.	Explain the principle of operation of any one type of single phase induction motor with 	necessary diagrams.								                 					(8)

b.	Draw and explain the slip-torque characteristics of a typical 3-phase induction motor. Mark 	the starting torque and maximum torque regions on the diagram so drawn.			(7)

(OR)

23.	a.	A 200 kW, 400 V, separately excited DC motor runs at 600 rpm. It has 864 lap-connected 	conductors. The full load copper loss is 8 kW. Calculate the useful flux/pole.		(6)

b.	Explain the principle of operation of a synchronous motor.                            			(9)

24.	a.	Describe the working principle of permanent magnet type moving coil measuring 	instruments.									                						(6)

b.	Explain the construction and working principle of induction type energy meter.   	(9)

(OR)

25.	a.	Draw the wiring diagram of a fluorescent lamp. Describe its working.	                 	(7)

b.	What is the necessity of earthing in domestic wiring? Discuss in brief the various methods 	of earthing employed. 							                 					(8)