Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ELECTRIC CIRCUIT ANALYSIS						      	       Time: 3 hours

Subject Code: EE201						       		               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The symbol of an independent current source is _____________.

2.	Two resistors of resistances R1 and R2 are connected in parallel. The total current is I amperes. 	The current I1 through R1 is ______________.

3.	Power factor is the cosine of the angle between ______________ and _______________.

4.	In a star-connected circuit, the line voltage is _________ times the phase voltage.

5.	Name the basic circuit law upon which the mesh method of circuit analysis is based.

6.	Transformers in which current and voltage ratios that do not depend on load or frequency are said to be __________________.

7.	Superposition theorem is applicable only to _____________ circuits.

8.	Norton’s equivalent circuit consists of ______________.

9.	The forced response of a sinusoidally excited AC circuit is also known as _________________.

10.	In one time constant, a capacitor discharges ______________ % of its initial charge.

PART – B  (5 x 3 = 15 MARKS)

11.	What are lumped and distributed elements? Give examples.

12.	Define Bandwidth and Q-factor.

13.	Distinguish between mesh and loop of an electrical network. Give an example.

14.	State Millman’s theorem for ‘n’ number of voltage sources and ‘n’ number of current sources.

15.	In the DC response analysis of an RL circuit, write the expressions for the transient current, transient voltage across resistor, and transient voltage across inductor. Draw the corresponding responses.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	State and explain Kirchoff’s laws.						                 				(5)

b.   	Using source transformation, find the power delivered by the 50 V voltage source in the 	circuit shown in figure.										                          	(10)

(OR)

17.	a.	Derive expressions for star connected resistances in terms of delta connected resistances.																						(10)

b.  	Explain the concept of voltage division and current division in electric circuits.	     	(5)

18.	a.    Explain about phasor relationships for circuit elements.			                 		(5)

b.   	A 3-phase, balanced delta-connected load of (16 + j12)Ω is connected across a 400V,        3-phase supply. Determine the phase current, line current, power factor, power, reactive VA 	and total VA.					         	                          					(10)

(OR)

[P.T.O]

19.	a.	Derive an expression for resonant frequency of a series RLC circuit.	                 	(6)

b.	Two wattmeter method is used to measure power in a 3-phase load. The wattmeter readings are 400 W and –35 W. Calculate 	i. 	Total active power	ii. Power factor

iii.	Reactive power.  		    						                 					(9)

20.  In the circuit shown in the figure, find the loop currents I1, I2, I3, and the power delivered to the 	load.

(OR)

21.  a.   	Write the node voltage equations and determine the currents in each branch for the network 		shown in the figure.							             							(10)

b.	Derive an expression for coefficient of coupling in a magnetically coupled circuit.  	(5)

22.	a.   	Verify the reciprocity theorem in the circuit shown in the figure.		         		(10)

b.   State and explain substitution theorem.		                                                   		(5)

(OR)

23.	Determine the maximum power delivered to the load in the circuit shown in the figure.

24.	For an RLC series circuit, with a step input, determine the current response i(t) for over 	damped, critical damped and under damped cases.

(OR)

25.	Explain the sinusoidal response analysis of an RC circuit.