Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: NETWORK ANALYSIS AND SYNTHESIS				       	       Time: 3 hours

Subject Code: EE203				       					             	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Name any two network functions.

2.	Define a network function.

3.	What is immittance?

4.	Define frequency response.

5.	If a graph has N nodes it will have ______ twigs of the tree.

6.	Inverse of impedance is _______.

7.	Write the condition for symmetry in a two port network.

8.	Which parameter is used to represent the equivalent circuit of a BJT?

9.	How will you identify the ‘LC’ network from its network function?

10.	Name the element that is used to represent the pole at infinity in Foster’s first form.

PART – B (5 x 3 = 15 MARKS)

11.	Write the necessary conditions for driving point function.

12.	Obtain the magnitude and phase response of the function F (jω)  = (jω)2.

13.	Write the properties of a Tree.

14.	The Z parameters of a two port network are Z11 = 10Ω, Z22=20Ω, Z12 = Z21 = 5Ω, find the ABCD parameters.

15.	Test whether the polynomial P(s) = s4 + s3 + 5s2 + 3s + 4 is Hurwitz.

PART – C (5 x 15 = 75 MARKS)

16.	For the given network function, draw the pole –zero diagram and hence obtain the time domain response i(t).

I (s) =5s(s+1) (s2 + 4s+8).

(OR)

17.	Obtain the transform impedance of the network shown in the figure below.

[P.T.O]

18.	Find the amplitude and phase response from the poles and zeros of the network function given by:

N(s)=2s (s2 + 4s+5)

(OR)

19.	A series RLC network consists of  R=50 Ω, L=0.2H and C=10µF with an applied voltage of 20 V. Determine the resonant frequency. Find the Q factor of the circuit. Compute the lower and upper frequency limits and also find the bandwidth of the circuit.

20.	In the given network shown in the figure below obtain values of branch currents and branch voltages by the current variables method.

(OR)

21.	a.	Write short notes on V – shift and I – shift techniques.                                              	(6)

b.	i.	For the circuit shown in the figure below draw the oriented graph.				(9)

ii.	Select a tree.

iii.   Write a tie-set schedule.

[P.T.O]

22.	The hybrid parameters of a two-port network shown in the figure are h11=1 K; h12=0.003; h21=100; h22=50 µ mho. Find V2 and Z parameters of the network.

(OR)

23.	a.	Design an m-derived low pass with filter cut-off frequency of 1 KHz, design impedance of       	400 Ω, and resonant frequency 1100 Hz.                                                                       (8)

b.	Design an m-derived high pass filter with a cut-off frequency of 10 KHz, design impedance of 500 Ω, and m=0.4.												(7)

24.	Find the first and second Cauer form networks for the given function.

Z(s)=2s3+8ss2+1

(OR)

25.	Find the two Foster realizations of the given impedance function.

Z(s)=3(s2+1) (s2+16)s(s2+9)