Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ELECTRICAL TECHNOLOGY							       Time: 3 hours

Subject Code: EE205					       		        		       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Write down any two main parts of a DC machine.

2.	What is the condition for maximum power developed in a DC motor?

3.	The transformer works without change in frequency – True/False?

4.	Mention any one special type of transformer.

5.	Can a DC generator be converted into an alternator?

6.	Write any one application of a synchronous motor.

7.	In a 3-phase induction motor, the rotor field rotates at synchronous speed w.r.t._________.

8.	What type of starter is used to a start slip-ring induction motor?

9.	Single-phase induction motors are not self-starting – True/False?

10.	Define step angle.

PART – B  (5 x 3 = 15 MARKS)

11.	With a schematic diagram, explain the working of a DC shunt motor.

12.	What is a servostabilizer?

13.	Write down the conditions for parallel operation of two alternators.

14.	Explain any one speed control method of a 3-phase induction motor.

15.	What is the function of a Relay?

PART – C  (5 x 15 = 75 MARKS)

16.	Draw the schematic diagrams and explain the working principle of different types of DC generators based on their excitation.

(OR)

17.	Explain the different methods of speed control of a DC shunt motor? Explain with neat circuit 	diagrams.

18.	a.	Derive the EMF equation of single-phase transformer.		                         			(7)

b.	Explain the construction of different types of transformers with diagrams.          			(8)

(OR)

19.	A 3 kVA, 230/230 V, 50 Hz transformer gave the following results: O.C. test: 230 V, 2 A, 100 W;  S.C. 	test: 15 V, 13 A, 120 W. Determine the regulation and efficiency at 	full load 0.8 p.f. lagging.

20.	Explain the constructional details and working principle of different types of three-phase alternators with neat diagrams.

(OR)

21.	Draw and explain the V-curves and inverted V-curves of a synchronous motor.

22.	Explain the constructional details and principle of operation of different types of three-phase I.M. with neat diagrams.

(OR)

23.	Explain the different speed control methods of phase wound 3-phase induction motor.

24.	Describe the working of variable reluctance stepper motor with a neat diagram.

(OR)

25.	Describe the principle of operation of a shaded-pole single phase induction motor with a circuit and phasor diagram.