Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MICROPROCESSORS AND MICROCONTROLLERS  	       Time: 3 hours

Subject Code: EE238						       		        		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Write the bit manipulation instructions in 8085 instruction set.

2.	State the function of ALE signal in 8085.

3.	List out the flags in 8086.

4.	What are the operating modes in 8086?

5.	Give the control word format for simple I/O mode of 8255.

6.	Name the two modes of operation of a DMA controller.

7.	List out the different types of ADCs.

8.	Differentiate hardware and software interrupts.

9.	Compare microprocessors with microcontrollers.

10.	What is the function of TCON register in 8051?

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the timing diagram for the instruction ADD M.

12.	Discuss the various addressing modes of 8086.

13.	Explain the need of DMA.

14.	Find the content of the accumulator after execution of the following 8085 assembly code.

MVI A, 05H;

MVI B, 05H;

LOOP1:	ADD B;

DCR B;

JNZ  LOOP1;

ADI 03H;

HLT;

15.	Explain the instruction; SWAP A in 8051 with an example.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Draw the internal architecture of 8085 IC and explain each block.					(8)

b.	Write an 8085 Assemble Language Program to add an array of 8 bit data. The size of the 			array is ‘n’.																	(7)

(OR)

17.	a.	Explain the different addressing modes in 8085.									(8)

b.	Explain the function of a stack pointer in 8085.									(7)

18.	a.	Describe the architecture of 8086 with neat diagrams.								(8)

b.	Write an Assemble Language Program in 8086 to find the largest number in a given array of ‘n’ numbers.																(7)

(OR)

19.	a.	Draw the memory organization in 8086 and explain.								(8)

b.	Discuss the instruction set of 8086 with examples.								(7)

[P.T.O]

20.	Illustrate the internal structure of a Programmable Peripheral Interface IC with neat diagrams.

(OR)

21.	Draw the Block diagram of a Programmable Interrupt Controller IC  and explain the functions of each block.

22.	Explain the concepts of interfacing a DC motor with 8085 using a neat diagram and assembly program.

(OR)

23.	Draw the schematic of the traffic light control using 8085. Also develop the algorithm and assembly program.

24.	a.	Describe the architecture of 8051 with a neat diagram.							(8)

b.	Discuss about the timers of 8051.												(7)

(OR)

25.	Discuss in detail about stepper motor interfacing with 8051.