Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ELECTRIC CIRCUITS									       Time: 3 hours

Subject Code: EE245						       		       		       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Write down any two circuit elements.

2.	What is the condition to apply Ohm’s law?

3.	Define form factor.

4.	What is the RMS value for sine wave?

5.	Which law is used in nodal analysis?

6.	What is the voltage level of reference node?

7.	Draw the Thevenin’s equivalent circuit.

8.	What is the condition for maximum power transfer in a DC system?

9.	What is the condition for resonance?

10.	Define quality factor.

PART – B  (5 x 3 = 15 MARKS)

11.	State and explain Kirchhoff’s current law.

12.	Draw the power triangle diagram for a lagging circuit.

13.	Explain loop analysis.

14.	State and explain superposition theorem.

15.	Derive an expression for resonance frequency in an RLC series circuit.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	With circuit diagrams, explain dependent and independent sources.                    			(5)

b.	Two resistances R1 and R2 connected in parallel take a total current of It. Obtain expressions for the currents in the individual resistances. If R1 = 1.95 Ω, R2 = 0.05 Ω and   It = 50A, find I1 and I2.                                                                                       																			(10)

(OR)

17.	In the circuit shown in figure find:		a.	the equivalent resistance of the circuit	 	b. 	the total current 	c.	the current in the resistor between points B and C. The applied voltage is 200 V. All the values are in Ohms.

18.	A series circuit has R=10Ω, L=50mh and C=100µf and is supplied with 200V, 50Hz. Find:

a.	the impedance	b.	the current 	c.	the power	d.	the power factor	e.	the  voltage drops across each element.

(OR)

19.	In an AC circuit consisting of two elements in series, the equations for voltage and current are   e = 180 sin 314t and I = 28.4 sin (314t – π/3). Calculate:		a.	the effective voltage and current		b. the frequency	c.	the power factor		d.	the power 	e.	the values of the circuit constants.

[P.T.O]

20.	Find the currents IA and IB for the circuit shown in figure, by nodal method.

(OR)

21.	Calculate the currents in various resistors in the network shown in the figure using mesh 	analysis.

22.	Find the current through and the voltage across 4 ohm resistance in the circuit shown in the figure, by using Thevenin’s theorem.

(OR)

23.	Find the current through and the voltage across the 4 ohm resistance in the circuit shown in the figure by the principle of superposition.

24.	A series RLC circuit has R = 5 Ohms, L = 40 mH and C = 1 μF. Calculate:

a.	the Q of the circuit				b.	the separation between half power frequencies

c.	the 	resonant frequency 		d.	the half power frequencies f1 and f2.

(OR)

25.	For a two branch parallel circuit, RL = 15 Ω, RC = 30 Ω, XC = 30 Ω, E = 120 V and f = 60 Hz. For the condition of resonance, calculate:

a.	the two values of L 			b.	the two values of total current.