Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: NETWORK THEORY									       	       Time: 3 hours

Subject Code: EE246				       					              	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	If in an acb phase sequence, Van = 100 ∟− 20◦, then Vcn is:

2.	When a Y-connected load is supplied by voltages in abc phase sequence, the line voltages 	______ (lag / lead) the corresponding phase voltages by 30◦.

3.	The coefficient of coupling for two coils having L1 = 2 H, L2 = 8 H, M = 3 H is: _________.

4.	The _______________ (linear / Ideal / autotransformer ) transformers can be used as an isolation device

5.	An RL circuit has R = 2Ω  and L = 4 H. The time needed for the inductor current to reach 40 percent of its steady-state value is __________.

6.	The initial value of f (t) with transform F(s) = (s + 1) / ( (s + 2)(s + 3)) is: __________.

7.	For two-port reciprocal network the h21 = h12. (True / False)

8.	The difference between the half-power frequencies is called the _____________ bandwidth / cutoff frequency.

9.	Is the polynomial P(S) = S4+2S3+2S2+6S+10 a Hurwitz polynomial?

10.	Check the positive realness of the function Z(s) = (s+3) / (s+1).

PART – B (5 x 3 = 15 MARKS)

11.	A balanced Y-connected load with a phase resistance of 40 Ω and a reactance of 25 Ω is supplied by a balanced, positive sequence ∆ connected source with a line voltage of 210 V. Calculate the phase currents. Use Vab as reference.

12.	What are the three important properties of an ideal transformer?

13.	The coil of a certain relay is operated by a 12-V battery. If the coil has a resistance of 150 Ω and an inductance of 30 mH and the current needed to pull in is 50 mA, calculate the relay delay time.

14.	Design a symmetrical lattice attenuator to have characteristic impedance of 800 Ω and attenuation of 20 dB.

15.	What are the properties of a positive real function?

PART – C (5 x 15 = 75 MARKS)

16.	For the unbalanced circuit in the figure find: 		a.	the line currents	b.	the total complex power absorbed by the load	c.	the total complex power supplied by the source.

[P.T.O]

(OR)

17.	The two-wattmeter method produces wattmeter readings P1 = 1560 W and P2 = 2100 W when  	connected to a delta-connected load. If the line voltage is 220 V, Calculate:

a.	the per-phase average power	b.	the per phase reactive power	c.	the power factor

d.	the phase impedance.

18.	Derive the equivalent inductance of two parallel connected coupled coils.

(OR)

19.	For a single tuned circuit, derive:

a. 	Mutual inductance	b. 	Maximum output voltage 	c. 	maximum amplification.

20.	The RLC series circuit of the figure below has R=20Ω, L=0.05H and C=20μF with 100 V constant source. Find 	the transient current and assume the switch S is closed at t=0.

(OR)

21.	Determine the transfer function H(s) = Vo(s)/Io(s) of the circuit in the figure shown below.

22.	Find the transmission parameters for the circuit in the figure using interconnection of two networks concept.

(OR)

23.	a.	Design a low pass filter (T) having a cutoff frequency of 2 kHz to operate with a 	terminated load resistance of 500 Ω.

b. 	Design an m-derived low pass filter (only T section) having cut off frequency of 1 kHz, design impedance of 400 Ω, and the resonant frequency 1100 Hz.

24.	The driving point impedance of a one port reactive network is given by Z(S) = {5 (S2+4) (S2+25)} / { S (S2+16) }. Obtain the first and second Foster networks.

(OR)

25.	Find the two Cauer realizations of a driving point function given by Z(S) = (10S4+12S2+1) / (2S3+2S).