Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DC MACHINES AND TRANSFORMERS					       Time: 3 hours

Subject Code: EE250			       						        	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is separate excitation?

2.	What is commutation?

3.	Why should a DC series motor not be started without load?

4.	What are the advantages of Swinburne’s test?

5.	Why is the efficiency of a transformer maximum?

6.	What is the condition for maximum efficiency of a transformer?

7.	What is the objective of conducting polarity test?

8.	What is the use of an autotransformer?

9.	Why is star-star connection considered to be most economical for small high voltage transformers?

10.	Which type of connection is suitable for three phase to two phase transformation?

PART – B (5 x 3 = 15 MARKS)

11.	List out the functions of interpoles and compensating windings.

12.	What is the change incorporated in four point starter when compared with three point starter?

13.	What is all day efficiency?

14.	List out the conditions to be satisfied for parallel operation of two single phase transformers.

15.	Why should the secondary of a current transformer not be kept open?

PART – C (5 x 15 = 75 MARKS)

16.	Explain the principle of operation and constructional details of a DC generator with necessary sketches.

(OR)

17.	a.	Derive the EMF equation of a DC generator. 									(7)

b.	Draw and explain the no-load saturation characteristics of separately and self excited DC generator. 																	(8)

18.	a.	Explain the Ward-Leonard method of speed control. Also list down the advantages. 	(8)

b.	Discuss in detail about the reverse current braking and regenerative braking applied to DC 	shunt motors. 																(7)

(OR)

19.	a.	Draw and explain the electrical and mechanical characteristics of DC shunt and series 	motors. 																		(8)

b.	A 230 volt DC series motor takes 30 amperes for giving its rated output at 1500 rpm. Its resistance is 0.15 ohms. Calculate the resistance to be added to obtain rated torque at:

i.	starting 		ii.	 at 1200 RPM.   											(7)

20.	a.	Explain the working of transformer on load condition with necessary phasor diagrams. (8)

b.	Explain the Kapp regulation diagram.  											(7)

[P.T.O]

(OR)

21.	a.	A 40 KVA, 2400/230 volt, 50 Hz transformer has primary winding resistance of 0.15 ohms 	and leakage reactance of 0.20 ohms. The secondary winding has 0.025 Ohms and the 	leakage reactance is 0.011 ohms. Determine the equivalent winding resistance, reactance 	and impedance referred to the:

i.	high voltage side 		ii.	low voltage side. 									(7)

b.	Starting from the basics, draw the approximate equivalent circuit of a transformer.	(8)

22.	a.	Explain the method of conducting Sumpner’s test on transformers.					(7)

b.	Discuss the method of paralleling two single phase transformers with explanation regarding validation of conditions. 												(8)

(OR)

23.	a.	Explain the working of an auto-transformer and prove that saving of copper increases with 	transformation ratio approaching unity. 											(8)

b.	Two single phase transformers A and B rated at 300 KVA each are operated in parallel on both sides. Percentage impedances for both transformers A and B are (1.1 +   j5.5) and (1.4+ j4.2) respectively. Determine the load shared by each transformer when the total load is 600 KVA at 0.9 power factor lagging. 										(7)

24.	Explain the constructional details of three phase transformers with neat sketches. Also discuss about delta-delta connection of transformer.

(OR)

25.	Write short notes on the following:

a.	Potential transformers. 														(7)

b.	Scott connection of transformers. 												(8)