Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: INDUCTION AND SYNCHRONOUS MACHINES		       	       Time: 3 hours

Subject Code: EE251		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Skewing of Induction motor increases starting torque. True or False.

2.	What do you mean by fractional slip in Induction Motor?

3.	How can we reverse the rotation of 1-phase, split-phase motor?

4.	Single phase motor is self starting. Comment on the statement.

5.	Why is the EMF method called as pessimistic method?

6.	How does armature winding in alternators differ from those used in a DC machine?

7.	When is a synchronous motor said to be under excited?

8.	Define pull-out torque in synchronous motor.

9.	Define voltage regulation.

10.	Draw power angle characteristic curve of a salient pole machine.

PART – B (5 x 3 = 15 MARKS)

11.	Enumerate the possible reasons if a three phase induction machine fails to start.

12.	Draw the equivalent circuit of a single phase induction motor.

13.	Calculate the distribution factor for a 36 slot, 4 pole, single layer three phase winding.

14.	Draw the V-curves of the synchronous motor.

15.	Draw the phasor diagram for a salient pole synchronous machine.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Derive the condition for maximum torque of three phase induction motor under running 	condition.																	(8)

b.	Explain with a relevant diagram, the construction and working of star – delta starter.	(7)

(OR)

17.	A 440 volt, 3-Φ, 50-Hz, 4-pole, Y-connected induction motor has a full load speed of 1425 rpm. The rotor has an impedance of (0.4 + j4) ohm and rotor/stator turn ratio of 0.8. Calculate: a.	full-load torque 	b.	rotor current and full-load rotor copper loss 	c.	power output if windage and fricition losses amount to 500 W 	d.	maximum torque and the speed at which it occurs 	e.	starting current 	f.	starting torque.

18.	a.	Explain the principle of working of Linear Induction Motor and give its properties.	   (8)

b.	The main and auxiliary winding impedance of a 50 Hz, capacitor start single-phase 	Induction Motor are (3 + j2.7) Ω and (7 + j3) Ω respectively. Determine the value of 	capacitor to be connected in series with the auxiliary winding to achieve a phase difference 	of α = 900 between the currents of the two windings at start.						   (7)

(OR)

[P.T.O]

19.	a.	Explain why the Single Phase Induction Motor is not self starting, with double field 				revolving theory.																(12)

b.	Give the advantages and applications of a capacitor – run motor.		 			(3)

20.	The following test results are obtained on a 6600-V alternator:

Open-circuit voltage	:	3100	4900	6600	7500	8300

Field current(amps)	:	16		25		37.5		50		70

A field current of 20 A is found necessary to circulate full-load current on short-circuit of the 	armature. Calculate by 	a.	the ampere-turn method 	b.	the synchronous impedance 	method, 	the full-load regulation at 0.8 p.f. (lag). Neglect resistance and leakage reactance. State 	the 	drawbacks of each of these methods.

(OR)

21.	a.	Derive the expression for power developed by a synchronous generator.	 			(6)

b.	Explain how three phase alternators are synchronized.				 			(9)

22.	A 1000-KVA, 11,000-V, 3-Φ, star-connected synchronous motor has an armature resistance 	and reactance per phase of 3.5 Ω and 40 Ω respectively. Determine the induced e.m.f and 	angular retardation of the rotor when fully loaded at:		a.	unity p.f. 	b.	0.8 p.f. 	lagging 		c.	 0.8 	p.f. leading.

(OR)

23.	a.	Explain hunting.																(4)

b.	Compare synchronous and induction machines.									(5)

c.	A 400-V, 7.46 kW, 3-phase synchronous motor has negligible armature resistance and synchronous reactance of 10 W/phase. Determine the minimum current and the corresponding induced e.m.f. for full-load conditions. Assume an efficiency of 85 %. (6)

24.	A 440 volt, 50 Hz, delta connected synchronous generator has a direct-axis reactance of 0.12 Ω 	and a quadrature-axis reactance of 0.075 Ω per phase. The armature resistance is negligible. 	The generator is supplying 1000 amps at 0.8 lagging power factor.

a.	Find the excitation EMF neglecting saliency and assuming Xs =Xd.

b.	Find the excitation EMF accounting the saliency.

Compare and comment on results of parts ‘a’ and ‘b’.

(OR)

25.	Describe the operation of a salient pole synchronous machine with the help of phasor diagram. Derive the expression for excitation voltage in both generating and motoring modes of operation.