Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ELECTRICAL MACHINE DESIGN						       Time: 3 hours

Subject Code: EE252						       		      		       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the main dimensions of a rotating machine?

2.	What is fringing?

3.	The number of segments in a commutator is equal to ___________.

4.	Write down the Pitchelmayer’s equation.

5.	The transformers used in sub-stations and generating stations are called __________.

6.	Write down the core area factor Kc for a square core.

7.	For a good overall design of a three phase induction machine, L/τ ratio is ___________.

8.	Where is the mush winding used?

9.	What is short circuit ratio?

10.	In alternators, when length of air gap is increased, then stability ____________.

PART – B  (5 x 3 = 15 MARKS)

11.	Define specific magnetic loading and specific electric loading.

12.	What is an Interpole and what is its purpose?

13.	Draw the cross section of different types of transformer cores.

14.	List the factors which affect the length of air gap in induction machines.

15.	Mention the uses of damper windings in synchronous machines.

PART – C  (5 x 15 = 75 MARKS)

16.	Calculate the ampere turns required for the air gap of a DC machine given the following data:

Gross core length = 40 cm, air gap length = 0.5 cm, number of ducts = 5, width of each          duct = 1.0 cm, slot pitch = 6.5 cm, average value of flux density in the air gap = 0.63 T. Field form factor = 0.7, Carter’s coefficient = 0.82 for opening/gap length = 1.0 and Carter’s coefficient = 0.72 for opening/gap length = 2.0.

(OR)

17.	a.	Discuss in detail about the unbalanced magnetic pull.								(7)

b.	Explain about the types of leakage flux.											(8)

18.	a.	Derive the output equation of a DC machine.										(10)

b.	Enumerate the procedure for shunt field design.									(5)

(OR)

19.	Determine the main dimensions of the armature core, number of conductors, and commutator

segments for a 350kW, 500V, 450 rpm, 6 pole shunt generator assuming a square pole face 	with pole arc 70% of the pole pitch. Assume the mean flux density to be 0.7T and ampere 	conductors per cm to be 280.

20.	a.	Derive the output equation of a single phase transformer. 							   (7)

b.	Explain how to estimate the no-load current of a three phase transformer. 			   (8)

[P.T.O]

(OR)

21.	Design a suitable cooling tank with cooling tubes for a 500 kVA, 6600/440V, 50Hz, 3 phase

transformer with the following data. Dimensions of the transformer are 100 cm height, 96 cm 	length and 47 cm breadth. Total losses = 7 kW. Allowable temperature rise for the tank walls 	is 350C. Tubes of 5 cm diameter are to be used. Determine the number of tubes required and 	their possible arrangement.

22.	Find the main dimensions of a 15kW, 3 phase, 400 V, 50 Hz, 2810 rpm squirrel cage induction motor having an efficiency of 0.88 and a full load power factor of 0.9. Assume Specific magnetic loading = 0.5 Wb/m2, Specific electric loading = 2000 A/m. Take the rotor peripheral speed as approximately 20 m/s at synchronous speed.

(OR)

23.	A 3 phase, 3000 volts, 260 kW, 50 Hz, 10 pole squirrel cage induction motor gave the following results during preliminary design. Internal diameter of the stator = 75 cm, Gross length of the stator = 35 cm, Number of stator slots = 120, Number of conductors per slot =10. Based on the above data calculate the following for the squirrel cage rotor:

a.	Total losses in rotor bars.      		b.		Losses in end rings.

c.	Equivalent resistance 	of the rotor.

24.	Determine the main dimensions of a 75 MVA, 13.8 kV, 50 Hz, 62.5 rpm, 3 phase star connected alternator. Also find the number of stator slots, conductors per slot, conductor area and work out the winding details. The peripheral speed should be less than 40 m/s. Assume average gap density as 0.65 Wb/m2, Specific electric loading as 40,000 AC/m and current density as 4 amp/ mm2.

(OR)

25.	The following information has been obtained during the preliminary design of a 3 phase        500 kVA, 6.6 kV, 12 pole, 500 rpm, star connected salient pole alternator. Stator            diameter = 1.3 m, gross length of stator = 0.21 m, air gap flux per pole = 0.0404 Wb. Based on the above information, design the field system of the alternator giving the following details:

a.	Length of the air gap.

b.	Diameter of the rotor at the air gap surface.

c.	Dimension of the pole.