Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: POWER ELECTRONICS								      	       Time: 3 hours

Subject Code: EE254					       		        		       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Power MOSFET is a _____________ controlled device.

2.	What is a snubber circuit?

3.	Define delay angle.

4.	Mention the types of commutation.

5.	Write the expression for the average output voltage of a step-up chopper.

6.	Define cycloconverters.

7.	State any two applications of current source inverter.

8.	Is forced commutation method used in inverters?

9.	Why do you need isolation between gating circuits and power circuits?

10.	Which power converters are used for HVDC transmissions?

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the characteristics of TRIAC and mark the various regions of operation.

12.	Bring out the significance of a free-wheeling diode.

13.	Define current limit control.

14.	Define PWM control of inverter and mention its advantages.

15.	Write briefly about types of HVDC links.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Compare Power MOSFET and Power BJT.										(5)

b.	Discuss the static and switching operation of a Thyristor.							(10)

(OR)

17.	a.	Draw and explain the reverse recovery characteristics of a power diode.				(7)

b.	Give the basic structure, characteristics and explain the working principle of IGBT.	(8)

18.	Draw and explain the operation of a single-phase full converter bridge rectifier with RLE load with necessary waveforms. Also, derive the average and RMS output voltage of the rectifier.

(OR)

19.	Explain the operation of a three phase fully controlled six pulse converter with a neat circuit diagram and waveforms.

20.	Describe the operation of the single-phase AC to AC full wave voltage controller for a resistive (R) load with necessary circuits and waveforms. Derive the expression for the RMS load voltage and RMS load current.

(OR)

21.	Discuss the following in detail:

a. 	Type-C chopper.						 										(7)

b. 	Four quadrant chopper.														(8)

[P.T.O]

22.	Discuss the principle of working of a three-phase bridge inverter with an appropriate circuit diagram. Draw phase and line voltage waveforms on the assumption that each thyristor conducts for 180 and the resistive load is star-connected. The sequence of firing of various SCRs should also be indicated in the diagram.

(OR)

23.	a.	Compare Voltage Source Inverter and Current Source Inverter.					(5)

b.	What is Pulse Width Modulation? Discuss any two of its techniques with neat diagrams.																						(10)

24.	Explain the operation of any one gating signal generation circuit for a single-phase full wave controlled rectifier with a firing angle control. Draw the necessary waveforms.

(OR)

25.	Draw the block diagram of various types of UPS and explain the functions of each block in them.